Pennsylvania’s Comprehensive Plan for  Mine Reclamation

DRAFT  August, 1996

I.	Introduction

Pennsylvania has a history of coal mining beginning in the 1800's.  Coal from Pennsylvania fueled the industrial growth of the country and provided the energy to fight two world wars.  But this legacy has left numerous scars on the Pennsylvania landscape.  There are over 250,000 acres of abandoned surface mines with dangerous highwalls and water filled pits.  About 2,400 miles of streams do not meet water quality standards because of drainage from abandoned mines.  There are uncounted households without adequate water supply due to past mining practices.  Mine fires and subsidence plague many Pennsylvania communities.

Pennsylvania long  has been leader in reclaiming abandoned mine lands.  In the early 60's, a bond issue authorized $200M to solve the problems of the past.  Operation  Scarlift, as it was known, completed over 500 stream pollution abatement projects to the tune of  $75,000,000.  Seventy five underground mine fires were extinguished at a cost of $24,000,000 and over 150 areas subjected to mine subsidence were stabilized at a cost of $26,000,000.  Air pollution at 30 burning refuse banks was controlled for a cost of

$16,000,000.

The reclamation of abandoned mine lands including the abatement of mine drainage is essential to improving the environment and the well-being of the citizens of  Pennsylvania.  In the past, the selection of reclamation projects was somewhat random and based on the requirements of the funding source.  During the "Operation Scarlift" days,  the bond issue legislation focused attention on restoring streams impacted by acid mine drainage, on extinguishing mine fires and on controlling mine subsidence.  When federal funding became available from Title IV of the Surface Mining Control and Reclamation Act (SMCRA), OSM focused attention on Priority 1 and 2 health and safety problems rather than general welfare problems.  Mine drainage abatement was not considered a priority 1 or 2 problem in most cases.  Revisions to the federal policy now consider general welfare problems that are in proximity to populated areas or have adverse economic impacts as potential priority 2 problems.  While these changes to the federal law give more flexibility in expending the SMCRA funds, projects continue to be selected at random.

More recently, grassroots organizations have expressed the need to improve the quality of streams polluted by acid mine drainage.  While these organizations  emphasize  acid mine drainage treatment and abatement, they have taken an areal approach focusing on watershed cleanup.  Rehabilitation plans are developed addressing the watershed as a whole.  The Department, also,  has begun to take an areal approach looking at water supplies impacted by mining on a areal basis.

In order to  effectively plan for abandoned mine reclamation and to properly balance expenditures between competing priorities,  a comprehensive approach is needed.

II.	Purpose for This Plan

The purpose of this plan is to establish a frame-work for organizing reclamation efforts, for coordinating among those involved in reclamation activities, for prioritizing expenditures and for  decision-making.  The Plan is a guide; as such, it is not an absolute under which every action must fall.  The Plan is intended to help those involved in reclamation activities focus on similar, achievable goals.  The Plan is intended to be a Pennsylvania Plan, not the "property" of any one institution or group.

III.	Guiding Principles

In developing and implementing a comprehensive plan for mine reclamation, the resources (both human and financial) of the participants must be coordinated to insure cost-effective results.  In addition, resources are limited requiring priorities to be set.  The following set of principles are intended to guide (nothing is absolute) decision making.

- Partnerships between the DEP, watershed associations, local governments, environmental groups, other state agencies, federal agencies and other groups organized to reclaim abandoned mine lands are essential to achieving reclamation and abating acid mine drainage in an efficient and effective manner.

- Partnerships between AML interests and active mine operators are important and essential in reclaiming abandoned mine lands.

- Priority for the development of AML reclamation or AMD abatement projects will be given to watersheds for which there is an approved watershed rehabilitation plan.

- Priority  for the use of  available moneys from federal and other sources will be given to projects that have other sources or means to fund the project or that need the funds to match other sources of funds.

- Priority for the use of available moneys from federal and other sources will be given to projects where there are institutional arrangements for long-term operation and maintenance costs.

- No plan is an absolute;  occasional deviations are to be expected.

IV. 	Goals

In addition to the principles that will be used to guide decision making, specific goals to be achieved by the comprehensive plan are set forth below.  These goals are to be reviewed, modified, expanded or changed periodically to insure the goals meet the needs on the time.

- To create a focus for the abandoned mine lands program that properly balance expenditures between competing reclamation needs -- water quality improvement, water supply replacement, hazard removal, health and safety concerns and the general welfare of the public.

- To develop partnerships involving local citizens, local government and other groups that promote abandoned mine reclamation.

- To develop long-term funding sources that would allow for long-term planning and long-term funding commitments.

- To develop an areal approach to reclamation planning that will result in reclamation and rehabilitation of an entire geographical area.

- To encourage the development and use of innovative technologies that reduce the cost of reclamation.

- To coordinate the activities of the Abandoned Mine Land program with the Mine Regulatory program so that active mine operators are encouraged to remine and reclaim where possible.

- To insure that property owners who allow the use of their lands for long-term treatment of abandoned mine drainage are not subject to personal or environmental liabilities because of the projects.

V.	Plan Elements

A.	 Inventory of Abandoned Mine Land Problems

In order to develop a comprehensive reclamation plan, the size and scope of the abandoned mine problem must be understood.  Over the years, many surveys have been done to inventory the problem and measure its intensity.  Operation Scarlift developed an inventory of AMD problems.  The Bureau of Abandoned Mine Reclamation maintains an “Abandoned Mine Land“ inventory that was developed in the late 70’s under SMCRA but has not been adequately updated. This inventory contains a comprehensive list and assessment of abandoned mine lands problems and serves as the basis for OSM approval of Title IV grant expenditures. The inventory, while fairly comprehensive, does not contain a complete list of acid mine drainage problems nor a complete assessment of water supply restoration needs.

The inventory is currently maintained on a mainframe computer but is not geographically based.  So, information cannot be removed from the system in a user friendly manner.  This inefficiency costs time and money and is a Bureau priority for enhancement.

Local groups including watershed associations, conservation districts and others have been collecting data on abandoned mine lands problems for many years.  This data is extremely important in defining the scope of the problem and needs to become part of the statewide inventory.

To support comprehensive planning, a Geographical Information System is needed to store and retrieve information in an efficient, useful manner.  Such a system should be available to all who are involved in addressing AML problems.

B.	An Areal Approach to AML Reclamation

The reclamation and rehabilitation of abandoned mine lands needs to focus on geographical areas that contain abandoned mine land problems.  These can be watersheds, political units, geologic units or other areal measures. The geographical focus will allow the restoration of an entire area rather than at a single site.  Some common areas for this focus would include:

- Watersheds

The use of a watershed as an areal boundary is appropriate when the major problem is the pollution of the watershed by drainage from abandoned mines.  In determining if a watershed should be given attention, the following criteria should be considered:

- The potential for water quality improvement in the watershed

- The potential for a state / federal / local partnership

- The existence of funding from other sources

- The potential for remining in the watershed

Watersheds meeting the criteria would be ripe for the development of a watershed rehabilitation plan.

2.  Areas experiencing loss or contamination of domestic water supplies.

The use of water company service areas as an areal boundary is appropriate when many residents are experiencing water loss or contamination due to past mining.  The following criteria will be used to select water supply development projects.

- The public health concerns where there is a lack of potable water

- The number of household involved

- The availability of an approved source of water

- An agreement by local government or public utility to assume operation and maintenance costs

3.  Other areas for focus

The reclamation of surface mined lands should also be addressed on an areal basis. In determining if an area should be selected, the following should be considered.

- Areas that are adjacent to or support public recreational areas.

- Areas that protect or enhance public water supply sources..

- Areas where past mining activities create an immediate threat to public health or safety  (mine fires, mine subsidence, mine wastes).

An areal focus, however, cannot be an absolute. There will need to be some flexibility to address site specific, high profile problems as they arise.

C.	Institutional Partnerships

As discussed earlier, the magnitude of the abandoned mine land problem in Pennsylvania is greater than any one institution can address in the foreseeable future.  Therefore, partnerships among public and private institutions are essential to accomplishing the goals of this comprehensive plan.

Partnerships can develop at any phase of in the planning process.  The leadership role among the partners is the most important decision the partners must make. The earlier partners establish a leader and define their individual roles, the more effective they will be.  For the most part, leadership should lie with a local organization where there is a strong, local support and commitment to long-term solutions.

There is no formula for developing partnerships.  Most likely a single institution will become interested in an area and bring other institutions into a partnership as needs and interest arise.

D.	The Rehabilitation Plan.

Areas, whether they be watersheds, political units or utility service areas, require the development of a comprehensive rehabilitation plan.  When dealing with water quality improvement, the plan must identify all of the environmental problems within the area not just those related to mining.  Alternative solutions to the mining related problems must be identified including a cost estimate for each alternative.  For non-mining related problems, the status of clean-up efforts must be discussed and referrals made to the appropriate regulatory body.

When dealing with water supply issues, the plan must identify all the water loss or contamination problem within the defined area.  Alternatives for addressing those problems must be discussed.  Similarly, for other areal designations, the objective behind making the designation must be fully addressed.

The development of the rehabilitation plan should be done under the leadership of the institutional partners.  Realistically, the plan will undergo several iterations as the partners bring their individual needs into the process.  Over time the plan will evolve into a comprehensive framework for addressing the abandoned mine land problem.

The implementation of the plan must be discussed in detail including the extent to which each of the partners will contribute to the implementation.  A time schedule for implementation would be desirable.

E.	A Long-Term Financing Plan

A proposal for financing the implementation of the rehabilitation plan will be essential to its success.  The financing plan should be realistic, recognizing the annual constraints of the various funding sources.  Where matching funds are needed, commitments from all the funding sources must be a part of the financing plan.