[25 PA. CODE CHS. 86–90]

Coal Mining Permitting and Performance Standards

DRAFT	SEPTEMBER 12, 1997

Annex A

TITLE 25. ENVIRONMENTAL PROTECTION

PART I. DEPARTMENT OF ENVIRONMENTAL PROTECTION

Subpart C. PROTECTION OF NATURAL RESOURCES

ARTICLE I. LAND RESOURCES

CHAPTER 86. SURFACE AND UNDERGROUND MINING OF COAL: GENERAL

Subchapter A. GENERAL PROVISIONS

§ 86.2.	Scope.

(a)	 This chapter specifies certain general procedures and rules for those persons who engage in coal mining activities. This chapter[,] together with Chapters 87–90 [specify] SPECIFIES the procedures and rules for those who engage in coal mining activities.

(b)	THIS CHAPTER TOGETHER WITH CHAPTERS 87–90 APPLIES TO ALL COAL MINING ACTIVITIES, WITH THE FOLLOWING EXCEPTIONS:

(1)	EXTRACTION OF COAL OR COAL REFUSE REMOVAL UNDER A GOVERNMENT-FINANCED RECLAMATION CONTRACT FOR THE PURPOSES OF SECTION 4.8 OF THE SURFACE MINING CONSERVATION AND RECLAMATION ACT (52 P. S. § 1396.4h).

(2)	EXTRACTION OF COAL [HIS] AS AN INCIDENTAL PART OF FEDERAL, STATE OR LOCAL GOVERNMENT FINANCED HIGHWAY CONSTRUCTION PROJECT.

(3)	THE RECLAMATION OF ABANDONED MINE LANDS NOT INVOLVING EXTRACTION OF COAL OR EXCESS SPOIL DISPOSAL UNDER A WRITTEN AGREEMENT WITH THE PROPERTY OWNER AND APPROVED BY THE DEPARTMENT.

(4)	ACTIVITIES EXEMPTED FROM REGULATION AS SURFACE MINING BY THE UNITED STATES OFFICE OF SURFACE MINING RECLAMATION AND ENFORCEMENT AT 30 CFR 707 (RELATING TO EXEMPTION FOR COAL EXTRACTION LIMITED TO GOVERNMENT FINANCED HIGHWAY OR OTHER CONSTRUCTION).

Subchapter B. PERMITS REVIEW, PUBLIC

PARTICIPATION AND APPROVAL, DISAPPROVAL

OF PERMIT APPLICATIONS AND PERMIT TERMS AND CONDITIONS

§ 86.37. Criteria for permit approval or denial.

(a)	A permit or revised permit application will not be approved unless the application affirmatively demonstrates and the Department finds, in writing, on the basis of the information in the application or from information otherwise available, which is documented in the approval, and made available to the applicant, that the following exist:

*        *        *        *        *

(4)	The assessment of the probable cumulative impacts of all anticipated coal mining in the general area on the hydrologic balance as described in § 87.69, § 88.49, § 89.36 or § 90.35 has been made by the Department, and the activities proposed under the application have been designed to prevent MATERIAL damage to the hydrologic balance [within and] outside the proposed permit area..

*        *        *        *        *

(6)	[The effect of the proposed coal mining activities on properties listed on or eligible for listing on the National Register of Historic Places has been taken into account by the Department.]    The proposed activities will not adversely affect any publicly owned parks or places included on [or eligible for inclusion on] the National Register of Historic Places, except as provided for in Subchapter D.    THE EFFECT OF THE PROPOSED COAL MINING ACTIVITIES ON PROPERTIES LISTED ON OR ELIGIBLE FOR LISTING ON THE NATIONAL REGISTER OF HISTORIC PLACES HAS BEEN TAKEN INTO ACCOUNT BY THE DEPARTMENT.  This finding may be supported in part by inclusion of appropriate permit conditions or operational plan changes to protect historic resources, or a documented decision that no additional protective measures are necessary.

*        *        *        *        *

§ 86.40. Permit terms.

*        *        *        *        *

(b)	 A permit shall terminate if the permittee has not begun the coal mining activities covered by the permit within 3 years of the issuance of the permit. However, the Department may grant reasonable extensions of time for commencement of these activities upon receipt of a written statement showing that the extensions of time are necessary if litigation precludes the commencement or threatens substantial economic loss to the permittee OR IF THERE ARE CONDITIONS BEYOND THE CONTROL AND WITHOUT THE FAULT OR NEGLIGENCE OF THE PERMITTEE. Requests for extensions shall be submitted to the Department prior to expiration of the permit.

*        *        *        *        *

MINIMUM REQUIREMENTS FOR LEGAL FINANCIAL COMPLIANCE AND RELATED INFORMATION

§ 86.64.    Right of entry.

(a)	An application shall contain a description of the documents upon which the applicant bases his legal right to enter and commence coal mining activities within the permit area and whether that right is the subject of pending court litigation. THE DESCRIPTION SHALL IDENTIFY THE DOCUMENTS BY TYPE AND DATE OF EXECUTION, IDENTIFY THE SPECIFIC LANDS TO WHICH THE DOCUMENT PERTAINS AND EXPLAIN THE LEGAL RIGHTS CLAIMED BY THE APPLICANT.

[(b)	 The application for a permit shall provide one of the following for lands within the permit area:

(1)	 A copy of the written consent of the current surface owner to the extraction of coal by surface mining methods.

(2)	A copy of the document of conveyance that expressly grants or reserves the right to extract the coal by surface mining methods and an abstract of title relating the documents to the current surface land owner.]

(b)	 WHERE THE PRIVATE MINERAL ESTATE TO BE MINED HAS BEEN SEVERED FROM THE PRIVATE SURFACE ESTATE, AN APPLICANT SHALL ALSO SUBMIT ONE OF THE FOLLOWING:

(1)	A COPY OF THE WRITTEN CONSENT OF THE SURFACE OWNER FOR THE EXTRACTION OF COAL BY SURFACE MINING METHODS.

(2)	 A COPY OF THE CONVEYANCE THAT EXPRESSLY GRANTS OR RESERVES THE RIGHT TO EXTRACT COAL BY SURFACE MINING METHODS.

(3)	 IF THE CONVEYANCE DOES NOT EXPRESSLY GRANT THE RIGHT TO EXTRACT THE COAL BY SURFACE MINING METHODS, DOCUMENTATION THAT UNDER PENNSYLVANIA LAW, THE APPLICANT HAS THE LEGAL AUTHORITY TO EXTRACT THE COAL BY THOSE METHODS.

(c)	 THIS SECTION WILL NOT BE CONSTRUED TO PROVIDE THE DEPARTMENT WITH THE AUTHORITY TO ADJUDICATE PROPERTY RIGHTS DISPUTES.

[(c)] (d) ***

*        *        *        *        *

(3)	THE REQUIREMENTS OF THIS SUBSECTION ARE IN ADDITION TO THE INFORMATION REQUIRED BY SUBSECTIONS (a) AND (b).

[(d)] (e) ***

[(e)] (f) The information required in [subsections (a) and (b)] THIS SECTION shall be made part of the permit application prior to approval of the bond under Subchapter F (relating to bonding and insurance requirements).

§ 86.70. Proof of publication.

An application shall contain [a statement from the newspaper] AN INTENT TO PUBLISH AND A COPY OF THE LANGUAGE TO APPEAR IN THE PUBLIC NOTICE DEMONSTRATING that the advertisement requirement of § 86.31(a) (relating to public notices of filing of permit applications) is in the process of being satisfied, and prior to the issuance of the permit, but no later than 4 weeks after the last date of advertisement, the applicant shall submit a copy of [advertisement] THE ADVERTISEMENTS as required by § 86.31(a) [and] OR the original notarized proof of publication to the Department.

Subchapter E. COAL EXPLORATION

§ 86.132. Definitions.

The following words and terms, when used in this subchapter, have the following meanings, unless the context clearly indicates otherwise:

*        *        *        *        *

Substantially disturb–For purposes of coal exploration, including, but not limited to, to have [an] A SIGNIFICANT impact upon land, air or water resources by activities such as blasting, mechanical excavation or altering coal or water exploratory holes or wells, construction of roads and other access routes, removal of topsoil or overburden and the placement of structures, excavated earth or other debris on the surface of land.

§ 86.133.    General requirements.

*        *        *        *        *

(e)	A person who intends to conduct coal exploration in which coal will be removed shall, prior to conducting the exploration, obtain a permit under this chapter. Prior to removal of coal, the Department may waive the requirements for the permit to enable the testing and analysis of coal properties, if [less than] 250 tons (226 METRIC TONS) or less is removed. The removal of MORE THAN 250 tons [or more] (226 METRIC TONS) of coal during coal exploration requires a permit under this chapter.

*        *        *        *        *

§ 86.134. Coal exploration performance and design standards.

The following performance standards are applicable to coal exploration which substantially disturbs the land surface:

*        *        *        *        *

[(2) The person who conducts coal exploration shall, to the extent practicable, measure important environmental characteristics of the exploration area during the operations to minimize environmental damage to the area and to provide supportive information for a permit application that person may submit under this chapter.

(3)] (2) [Vehicular travel on other than established graded and surfaced roads shall be limited by the person who conducts coal exploration to that absolutely necessary to conduct the exploration.] ROADS USED FOR COAL EXPLORATION SHALL COMPLY WITH THE FOLLOWING:

*        *        *        *        *

[(4)] (3) ***

[(5)] (4) ***

[(6)] (5) [Revegetation of areas disturbed by coal exploration shall be performed by the person who conducts the exploration, or the person's agent] ALL AREAS DISTURBED BY COAL EXPLORATION ACTIVITIES SHALL BE REVEGETATED IN A MANNER THAT ENCOURAGES PROMPT REVEGETATION AND RECOVERY OF A DIVERSE, EFFECTIVE AND PERMANENT VEGETATIVE COVER.

[(7)] (6) ***

*        *        *        *        *

[(8)] (7) ***

[(9)] (8) ***

*        *        *        *        *

[(10)] (9) ***

[(11)] (10) ***

[(12)] (11) ***

Subchapter F. BONDING AND INSURANCE REQUIREMENTS

RELEASE OF BONDS

§ 86.174.	Standards for release of bonds.

*        *        *        *        *

(b)	When the entire permit area or a portion of the permit area meets the following standards, Stage 2 reclamation has been achieved:

(1)	Topsoil has been replaced and revegetation has been SUCCESSFULLY established in accordance with the approved reclamation plan [and the standards for the success of revegetation are met].

(2) The reclaimed lands are not contributing suspended solids to stream flow or runoff outside the permit area in excess of [their] THE requirements of the acts, regulations thereunder or the permit.

*        *        *        *        *

CHAPTER 87. SURFACE MINING OF COAL

Subchapter A. GENERAL PROVISIONS

§ 87.1. Definitions.

The following words and terms, when used in this chapter, have the following meanings, unless the context clearly indicates otherwise:

*        *        *        *        *

Land use–Specific uses or management-related activities, rather than the vegetation or cover of the land. Land uses may be identified in combination when joint or seasonal uses occur. Changes of land use from one of the following categories to another shall be considered as a change to an alternative land use which is subject to approval by the Department. Land use may be defined as:

*        *        *        *        *

(X)	UNMANAGED NATURAL HABITAT. IDLE LAND WHICH DOES NOT REQUIRE A SPECIFIC MANAGEMENT PLAN AFTER THE RECLAMATION AND REVEGETATION HAVE BEEN ACCOMPLISHED.

*        *        *        *        *

Subchapter D. SURFACE COAL MINES: MINIMUM REQUIREMENTS FOR OPERATION AND RECLAMATION PLAN

§ 87.77. Protection of [public] [PUBLICLY OWNED] PUBLIC parks and historic places.

(a)	 For [public parks or historic places] PUBLICLY OWNED PARKS OR HISTORIC PLACES LISTED ON THE NATIONAL REGISTER OF HISTORIC PLACES that may be adversely affected by the proposed operations, each application shall describe the measures to be used to accomplish the following:

*        *        *        *        *

Subchapter E. SURFACE COAL MINES: MINIMUM ENVIRONMENTAL PROTECTION

PERFORMANCE STANDARDS

§ 87.93. Casing and sealing of drilled holes.

(a)	Each exploration hole, other drill or borehole, well or other exposed underground opening (except for holes solely drilled and used for blasting) shall be cased, sealed or otherwise managed as approved by the Department in order to:

*        *        *        *        *

(2)	[Prevent to the maximum extent possible] MINIMIZE disturbance to the prevailing hydrologic balance.

*        *        *        *        *

(e)	A solid barrier of undisturbed earth, 125 feet (38.1 METERS) in radius shall be maintained around all oil and gas wells, except in the case of one of the following:

*        *        *        *        *

(2)	The Department approves, in writing, a lesser distance, provided:

*        *        *        *        *

[(iii) The well operator agrees, in writing, to the lesser distance.]    (iii)    THE WELL OPERATOR OF AN OPERATING OIL OR GAS WELL AGREES, IN WRITING, TO THE LESSER DISTANCE.

§ 87.97.    Topsoil: removal.

*        *        *        *        *

(c)	 If topsoil is less than 12 inches (30.48 CENTIMETERS), a 12-inch (30.48 CENTIMETERS) layer which includes the topsoil and the unconsolidated materials immediately below the topsoil shall be removed, segregated, conserved and replaced as the final surface soil layer. If the topsoil and the unconsolidated material measure lessthan 12 inches (30.48 CENTIMETERS), the topsoil and all unconsolidated material shall be removed, segregated, conserved and replaced as the final surface soil layer.

*        *        *        *        *

(f)	When approved by the Department in writing, other materials may be substituted for or used as a supplement to [subsoil] TOPSOIL if the operator demonstrates that the resulting soil medium is equal to or more suitable than [subsoil] TOPSOIL for sustaining vegetation and soil productivity. In making this demonstration, the Department may require chemical and physical analyses of the substituted material and [subsoil] TOPSOIL. These analyses may include determinations of pH, net acidity or alkalinity, phosphorus, potassium, texture class, field site trials or greenhouse tests, or other analyses as required by the Department.

§ 87.101.	Hydrologic balance: general requirements.

(a)	Surface mining activities shall be planned and conducted to [prevent, to the maximum extent possible, the] MINIMIZE disturbances to the prevailing hydrologic balance in the permit and adjacent areas AND TO PREVENT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA.    THE DEPARTMENT MAY REQUIRE ADDITIONAL PREVENTATIVE, REMEDIAL, OR MONITORING MEASURES TO ASSURE THAT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA IS PREVENTED.

*        *        *        *        *

§ 87.102.	Hydrologic balance: effluent standards.

(a)	Discharge of water. A person may not allow a discharge of water from an area disturbed by coal mining activities, including areas disturbed by mineral preparation, processing or handling facilities which exceeds the following groups of effluent criteria. The effluent limitations shall be applied under subsection  (b).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 |  | greater than 6.0; less than 9.0 |  |

alkalinity greater than acidity1

1 This parameter is applicable at all times

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |

alkalinity greater than acidity

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |

alkalinity greater than acidity

*        *        *        *        *

§ 87.106.	Hydrologic balance: sediment control measures.

Appropriate sediment control measures shall be designed, constructed and maintained using the best technology currently available to:

(1)	Prevent to the [maximum] extent possible contributions of sediment to streamflow or to runoff outside the affected area.

*        *        *        *        *

(3)	[Prevent] MINIMIZE erosion to the [maximum] extent possible.

*        *        *        *        *

§ 87.126. Use of explosives: public notice of blasting schedule.

(a)	Blasting schedule publication.

(1)	Each person who conducts surface mining activities shall publish a blasting schedule in a newspaper of general circulation in the locality of the proposed site, at least 10 days, but not more than [20] 30 days, before beginning a blasting program in which blasts that use more than 5 pounds of explosives or blasting agents are detonated.

*        *        *        *        *

§ 87.127. Use of explosives: surface blasting requirements.

*        *        *        *        *

(f)	Requirements for blasting are as follows:

*        *        *        *        *

(5)	Flyrock, including blasted material traveling along the ground, may not be cast from the blasting vicinity more than one-half the distance to the nearest dwelling or other occupied structure and in no case beyond the [line of property owned or leased by the permittee] PERMIT BOUNDARY, or beyond the area or regulated access required under subsection (d).

*        *        *        *        *

§ 87.138. Protection of fish, wildlife and related environmental values.

(a)	 A person conducting surface mining activities shall, TO THE EXTENT POSSIBLE using the best technology currently available:

(1)	[Prevent] MINIMIZE disturbances and adverse impacts of the activities on fish, wildlife and related environmental values, and achieve enhancement of the resources when practicable.

(2)	Locate and operate haul and access roads to [prevent] AVOID OR MINIMIZE impacts to fish and wildlife species or other species protected by State or Federal law.

(3)	[Prevent] AVOID disturbance to, ENHANCE WHERE PRACTICABLE, OR RESTORE, habitats of unusually high value for fish and wildlife.

*        *        *        *        *

(b)	 A person who conducts surface mining activities shall promptly report to the Department the presence in the permit area of threatened or endangered species under State or Federal laws of which that person becomes aware and which was not previously reported to the Department by that person. Upon notification, the Department will consult with the Game Commission or the Fish and Boat Commission and appropriate Federal fish and wildlife agencies and, after consultation, will identify whether, and under what conditions, the operator may proceed.

*        *        *        *        *

§ 87.144. Backfilling and grading: final slopes.

*        *        *        *        *

(c)	On approval by the Department in order to conserve soil moisture, ensure stability, and control erosion on final graded slopes, cut and fill terraces may be allowed if the terraces are compatible with the approved postmining land use and are substitutes for construction of lower grades on the reclaimed lands. [The terraces shall meet the following requirements:

(1)	The width of the individual terrace bench shall not be less than 20 feet.

(2)	 The vertical distance between terraces shall be as approved by the Department to prevent excessive erosion and to provide long-term stability.

(3)	The slope of the terrace outslope may not exceed 1v:2h–50%. Outslopes which exceed 1v:2h–50% may be approved, if they have a minimum static safety factor of more than 1.3, provide adequate control over erosion, and closely resemble the surface configuration of the land prior to mining. In no case may highwalls be left as part of terraces.

(4)	Culverts and underground rock drains may be used on the terrace only when approved by the Department.]

*        *        *        *        *

(f)	All final grading, preparation of overburden before replacement of topsoil, and placement of topsoil shall be [done along the contour to minimize subsequent erosion and instability. If the grading, preparation, or placement along the contour is hazardous to equipment operators, then grading, preparation or placement in a direction other than generally parallel to the contour may be used. In all cases grading, preparation or placement shall be] conducted in a manner which minimizes erosion and provides a surface for replacement of topsoil which will minimize slippage.

§ 87.146. Regrading or stabilizing rills and gullies.

[When a rill or gully deeper than 9 inches forms in areas that have been regraded and planted, the rill and gully shall be filled, graded or otherwise stabilized and the area reseeded or replanted under §§ 87.147–87.153, 87.155 and 87.156. The Department will specify that a rill or gully of lesser size be stabilized and the area reseeded or replanted if the rill or gully is disruptive to the approved postmining land use or may result in additional erosion and sedimentation. The regrading or stabilization of a rill or gully to include the reseeding or replanting shall be completed by the first normal period for favorable planting as established within § 87.148 (relating to revegetation: timing), except a rill or gully contributing to impacts outside the permit area or a rill or gully occurring after removal of erosion and sedimentation control structures which shall be stabilized immediately.]

(a)	EXPOSED SURFACE AREAS SHALL BE PROTECTED AND STABILIZED TO EFFECTIVELY CONTROL EROSION AND AIR POLLUTION ATTENDANT TO EROSION.

(b)	RILLS AND GULLIES, WHICH FORM IN AREAS THAT HAVE BEEN REGRADED AND TOPSOILED AND WHICH DO ONE OF THE FOLLOWING SHALL BE FILLED, REGRADED OR OTHERWISE STABILIZED:

(1)	DISRUPT THE APPROVED POSTMINING LAND USE OR THE REESTABLISHMENT OF THE VEGETATIVE COVER.

(2)	CAUSE OR CONTRIBUTE TO A VIOLATION OF WATER QUALITY STANDARDS FOR RECEIVING STREAMS.

(c)	FOR THE AREAS LISTED IN SUBSECTION (B), THE TOPSOIL SHALL BE REPLACED AND THE AREAS SHALL BE RESEEDED OR REPLANTED.

§ 87.159. Postmining land use.

*        *        *        *        *

(b)	The premining use of land to which the postmining land use is compared shall be determined by the following:

*        *        *        *        *

[(3)	The postmining land use for land that has received improper management shall be judged on the basis of the premining use of surrounding lands that have received proper management.

(4)	The postmining land use of land which was changed within 5 years of the beginning of mining shall be judged on the basis of the historic use of the land as well as its use immediately preceding mining.]

(c)	Alternative land uses may be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands and after determining that the following criteria are met:

*        *        *        *        *

(2)	[The owner of the surface requests in a notarized written statement that such alternative land use be approved.]    THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY ONE OR MORE OF THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(3)] (i) ***

[(4)] (ii) ***

[(5)] (iii) ***

[(6)	Plans for the postmining land use are designed and certified by a registered professional engineer in conformance with professional standards established to assure the stability, drainage and configuration necessary for the intended use of the site.]

[(7)] (3) ***

[(8)] (4) ***

[(9)] (5) ***

§ 87.160. Haul roads and access roads.

(a)	Haul roads and access roads shall be designed, constructed and maintained to [prevent, to the maximum extent possible,] CONTROL OR PREVENT erosion and [to prevent] contributions of sediment to streams or runoff outside the affected area; air and water pollution; damage to fish and wildlife or their habitat; and flooding and damage to public or private property. To ensure environmental protection appropriate for their planned duration and use, including consideration of the type and size of equipment used, the design and construction or reconstruction of roads shall incorporate appropriate limits for grade, width, surface materials, surface drainage control, culvert placement and culvert size, in accordance with current, prudent engineering practices and necessary design criteria established by the Department. Upon completion of the associated surface mining activities, the area disturbed by the road shall be restored in accordance with § 87.166 (relating to haul roads and access roads: restoration) unless retention of the road and its maintenance plan is approved as part of the postmining land use.

*        *        *        *        *

§ 87.166. Haul roads and access roads: restoration.

Unless the Department approves retention of a road as suitable for the approved postmining land use in accordance with § 87.159 (relating to postmining land use), [immediately] AS SOON AS PRACTICABLE after the road is no longer needed for the associated surface mining activities:

*        *        *        *        *

(4)	Roadbeds shall be ripped[, plowed and] OR scarified.

*        *        *        *        *

§ 87.173. Support facilities and utility installations.

(a)	Support facilities required for, or used incidentally to, the operation of the mine, including, but not limited to, mine buildings, coal loading facilities at or near the mine- site, coal storage facilities, equipment storage facilities, fan buildings, hoist buildings, preparation plants, sheds, shops and other buildings shall be [designed, constructed or reconstructed and located to prevent or control erosion and siltation, water pollution, and damage to public or private property. Support facilities shall be designed, constructed or reconstructed, maintained and used in a manner which prevents, to the maximum extent possible using the best technology currently available:

(1)	Damage to fish, wildlife and related environmental values.

(2)	Erosion and additional contributions of suspended solids to streamflow or runoff outside the permit area. Any such contributions may not be in excess of limitations of State or Federal law.] LOCATED, MAINTAINED AND USED IN A MANNER THAT DOES THE FOLLOWING:

(1)	PREVENTS OR CONTROLS EROSION AND SILTATION, WATER POLLUTION, AND DAMAGE TO PUBLIC OR PRIVATE PROPERTY.

(2)	TO THE EXTENT POSSIBLE USING THE BEST TECHNOLOGY CURRENTLY AVAILABLE:

(i)	MINIMIZES DAMAGE TO FISH, WILDLIFE AND RELATED ENVIRONMENTAL VALUES.

(ii)	MINIMIZES ADDITIONAL CONTRIBUTIONS OF SUSPENDED SOLIDS TO STREAMFLOW OR RUNOFF OUTSIDE THE PERMIT AREA. THESE CONTRIBUTIONS MAY NOT BE IN EXCESS OF LIMITATIONS OF STATE OR FEDERAL LAW.

*        *        *        *        *

§ 87.174. Steep slope operations.

*        *        *        *        *

[(g) Unlined or unprotected drainage channels may not be constructed on the backfilled areas unless approved by the Department as stable and not subject to erosion.]

§ 87.176. Auger mining.

*        *        *        *        *

[(d) Auger mining operations may not be conducted to the rise of the coal unless the person who conducts mining activities demonstrates to the satisfaction of the Department that drainage from the auger hole will not pose a threat of pollution to surface waters and will comply with § 87.102 (relating to hydrologic balance: effluent standards).]

[(e)] (d)	The Department will prohibit auger mining [if it determines that one of] UNLESS THE PERSON CONDUCTING THE SURFACE MINING ACTIVITIES DEMONSTRATES, the following [applies]:

(1)	Adverse water quality impacts [cannot] CAN be prevented or corrected.

(2)	Fill stability [cannot] CAN be achieved.

(3)	The [prohibition] AUGER MINING is necessary to maximize the utilization, recoverability or conservation of the solid fuel resources.

(4)	Subsidence resulting from auger mining [may] WILL NOT disturb or damage powerlines, pipelines, buildings or other facilities.

Subchapter F. SURFACE COAL MINES: MINIMUM REQUIREMENTS FOR REMINING AREAS WITH POLLUTIONAL DISCHARGES

§ 87.209.	Criteria and schedule for release of bonds on pollution abatement areas.

(a)	The Department will release up to [50] 60% of the amount of bond for the authorized pollution abatement area if the applicant demonstrates and the Department finds that:

*        *        *        *        *

(4)	The operator has not caused degradation of the baseline pollution load [for] AT ANY TIME DURING a period of [a minimum of] 6 months prior to the submittal of the request for bond release under this subsection and until the bond release is approved as shown by all ground and surface water monitoring conducted by the permittee under § 87.206(1) (relating to operational requirements) or conducted by the Department.

*        *        *        *        *

(b)	The Department will release an additional amount [not to exceed 35% of the amount of bond for the authorized pollution abatement area if the applicant demonstrates and the Department finds that] OF BOND FOR THE AUTHORIZED POLLUTION ABATEMENT AREA BUT RETAIN AN AMOUNT SUFFICIENT TO COVER THE COST TO THE DEPARTMENT OF REESTABLISHING VEGETATION IF COMPLETED BY A THIRD PARTY IF THE OPERATOR DEMONSTRATES AND THE DEPARTMENT FINDS THAT:

*        *        *        *        *

(3)	The operator has complied with one [either] of the following:

*        *        *        *        *

(ii)	Achieved the following:

(A)	At a minimum has not caused degradation of the baseline pollution load as shown by ground and surface water monitoring conducted by the operator or the Department for one of the following:

(I)	For a period of 12 months [from the date of initial bond release under subsection (a),] PRIOR TO THE DATE OF APPLICATION FOR BOND RELEASE AND UNTIL THE BOND RELEASE IS APPROVED UNDER SUBSECTION (b), if backfilling, final grading, drainage control, topsoiling, and establishment of revegetation to achieve the standard of success for revegetation [set forth] in § 87.205(a)(5) have been completed.

*        *        *        *        *

(c)	The Department will release the remaining portion of the amount of bond on the authorized pollution abatement area if the applicant demonstrates and the Department finds that:

*        *        *        *        *

(4)	The applicable liability period[, which shall be measured from the date of release of bond under subsection (b),] has expired under § 86.151 (relating to period of liability).

CHAPTER 88. ANTHRACITE COAL

Subchapter A. GENERAL PROVISIONS

PRELIMINARY PROVISIONS

§ 88.1.	Definitions.

The following words and terms, when used in this chapter, have the following meanings, unless the context clearly indicates otherwise:

*        *        *        *        *

Land use–Specific uses or management-related activities, rather than the vegetation or cover of the land. Land uses may be identified in combination when joint or seasonal uses occur. A change of land use from one of the following categories to another shall be considered as a change to an alternative land use which is subject to approval by the Department. The term is further defined as:

*        *        *        *        *

(x)	UNMANAGED NATURAL HABITAT. IDLE LAND WHICH DOES NOT REQUIRE A SPECIFIC MANAGEMENT PLAN AFTER THE RECLAMATION AND REVEGETATION HAVE BEEN ACCOMPLISHED.

*        *        *        *        *

§ 88.56. Protection of [public] [PUBLICLY OWNED] PUBLIC parks and historic places.

(a)	For [public parks or historic places] PUBLICLY OWNED PARKS OR HISTORIC PLACES LISTED ON THE NATIONAL REGISTER OF HISTORIC PLACES that may be adversely affected by the proposed operations, each application shall describe the measures to be used to accomplish the following:

*        *        *        *        *

Subchapter B. SURFACE ANTHRACITE COAL MINES: MINIMUM ENVIRONMENTAL PROTECTION PERFORMANCE STANDARDS

§ 88.83. Sealing of drilled holes: general requirements.

(a)	An exploration hole, other drill or borehole, well or other exposed opening (except for holes solely drilled and used for blasting), shall be sealed, backfilled or otherwise managed, as approved by the Department, in order to do the following:

*        *        *        *        *

(2)	[Prevent to the maximum extent possible] MINIMIZE disturbance to the prevailing hydrologic balance.

*        *        *        *        *

(e)	A solid barrier or undisturbed earth, 125 feet (38.1 METERS) in radius shall be maintained around all oil and gas wells, unless:

*        *        *        *        *

(2) The Department approves in writing a lesser distance provided:

*        *        *        *        *

[(iii) The well operator agrees in writing to the lesser distance.]    (iii)    THE WELL OPERATOR OF AN OPERATING OIL OR GAS WELL AGREES, IN WRITING, TO THE LESSER DISTANCE.

§ 88.91. Hydrologic balance: general requirements.

(a)	Surface mining activities shall be planned and conducted to [prevent, to the maximum extent possible,] MINIMIZE disturbances to the prevailing hydrologic balance in [both] the permit and adjacent areas AND TO PREVENT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA.    THE DEPARTMENT MAY REQUIRE ADDITIONAL PREVENTIVE, REMEDIAL, OR MONITORING MEASURES TO ASSURE THAT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA IS PREVENTED.    Where the area has been previously mined, the surface mining activity shall be planned and conducted to maximize the abatement of water pollution and the reclamation of the land.

*        *        *        *        *

§ 88.92. Hydrologic balance: effluent standards.

(a)	Groups of effluent criteria. A person may not allow a discharge of water from an area disturbed by mining activities which exceeds the following groups of effluent criteria. The effluent limitations shall be applied under subsection (b).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 | greater than 6.0; less than 9.0 |  |  |

alkalinity greater than acidity1

1 The parameter is applicable at all times.

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

*        *        *        *        *

§ 88.96. Hydrologic balance: sediment control measures.

Appropriate sediment control measures shall be designed, constructed and maintained to:

(1)	Prevent, to the [maximum] extent possible, contributions of sediment to streamflow or to runoff outside the affected area.

*        *        *        *        *

(3)	[Prevent] MINIMIZE erosion to the [maximum] extent possible.

*        *        *        *        *

§ 88.118. Backfilling and grading: final slopes.

*        *        *        *        *

(c)	Cut and fill terraces may be allowed on approval by the Department in order to conserve soil moisture, ensure stability and control erosion on final graded slopes, if the terraces are compatible with the approved postmining land use and are substitutes for construction of lower grades on the reclaimed lands. [The terraces shall meet the following requirements:

(1)	The vertical distance between terraces shall be as approved by the Department to prevent excessive erosion and to provide long-term stability.

(2)	The slope of the terrace outslope may not exceed 1v:2h–50%. Outslopes which exceed 1v:2h–50% may be approved, if they have a minimum static safety factor of more than 1.3, provide adequate control over erosion, and closely resemble the surface configuration of the land prior to mining. In no case may highwalls be left as part of a terrace face.

(3)	Culverts and underground rock drains may be used on the terrace only when approved by the Department.]

*        *        *        *        *

(f)	All final grading, preparation [or]OF overburden before replacement of soil or suitable vegetation support material shall be [done along the contour to minimize subsequent erosion and instability. If such grading, preparation or placement along the contour is hazardous to equipment operators, the grading, preparation, or placement in a direction other than generally parallel to the contour may be used. In all cases, grading, preparation or placement shall be] conducted in a manner which minimizes erosion and provides a surface for replacement of soil which will minimize slippage.

§ 88.133. Postmining land use.

(a)	All affected areas shall be restored in a timely manner to conditions that are capable of supporting the uses which they were capable of supporting before mining, or to higher or better uses achievable under criteria and procedures of this section and prior to the release of land from THE permit area in accordance with Chapter 86 [Subchapter E (relating to coal exploration)], SUBCHAPTER F (RELATING TO BONDING AND INSURANCE REQUIREMENTS).

(b)	The premining use of land to which the postmining land use is compared shall be determined by the following:

*        *        *        *        *

[(3)	The postmining land use for land that has received improper management shall be judged on the basis of the premining use of surrounding lands that have received proper management.

(4)	The postmining land use of land which was changed within 5 years of the beginning of mining shall be judged on the basis of the historic use of the land as well as its use immediately preceding mining.]

(c)	Alternative land uses will be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands and after determining that the following criteria are met:

*        *        *        *        *

(2)	[The owner of the surface requests in a notarized written statement that such alternative land use be approved.] THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY ONE OR MORE OF THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(3)] (i) ***

[(4)] (ii) ***

[(5)	Plans for the postmining land use are designed and certified by a registered professional engineer, in conformance with professional standards established to assure the stability, drainage and configuration necessary for the intended use of the site.]

[(6)] (3) ***

[(7)] (4) ***

[(8)] (5) ***

§ 88.138. Haul roads and access roads: general.

(a)	Haul roads and access roads shall be designed, constructed and maintained to [prevent to the maximum extent possible] CONTROL OR PREVENT erosion and [to prevent] contributions of sediment to streams or runoff outside the affected area; air and water pollution; damage to fish and wildlife or their habitat; flooding and damage to public or private property. Upon completion of the associated surface mining activities the area disturbed by the road shall be restored in accordance with § 88.144 (relating to haul roads and access roads: restoration) unless retention of the road and its maintenance plan is approved as part of the postmining land use.

*        *        *        *        *

§ 88.144. Haul roads and access roads: restoration.

Unless the Department approves retention of a road as suitable for the approved postmining land use in accordance with § 88.133 (relating to postmining land use), [immediately] AS SOON AS PRACTICABLE after the road is no longer needed for the associated surface mining activities:

*        *        *        *        *

Subchapter C. ANTHRACITE BANK REMOVAL AND RECLAMATION: MINIMUM ENVIRONMENTAL PROTECTION PERFORMANCE STANDARDS

§ 88.187. Hydrologic balance: effluent standards.

(a)	Groups of effluent criteria. A person may not allow a discharge of water from an area disturbed by mining activities which exceeds the following groups of effluent criteria. The effluent limitations shall be applied under subsection (b).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 |  | greater than 6.0; less than 9.0 |  |
| alkalinity greater than acidity1 |  |  |  |

1 The parameter is applicable at all times.

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

*        *        *        *        *

§ 88.191. Hydrologic balance: sediment control measures.

Appropriate sediment control measures shall be designed, constructed and maintained to:

(1)	Prevent, to the [maximum] extent possible, contributions of sediment to streamflow or to runoff outside the permit.

*        *        *        *        *

(3)	[Prevent] MINIMIZE erosion to the [maximum] extent possible.

*        *        *        *        *

§ 88.221.	Postmining land use.

*        *        *        *        *

(b)	The premining use of land to which the postmining land use is compared shall be determined by the following:

*        *        *        *        *

[(3)	The postmining land use for land that has received improper management shall be judged on the basis of the premining use of surrounding lands that have received proper management.

(4)	The postmining land use of land which was changed within 5 years of the beginning of mining shall be judged on the basis of the historic use of the land as well as its use immediately preceding mining.]

(c)	Alternative land uses shall be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands and after determining that the following criteria are met:

*        *        *        *        *

(2)	[The owner of the surface requests in a notarized written statement that alternative land use be approved.] THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY ONE OR MORE OF THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(3)] (i) ***

[(4)] (ii) ***

[(5)	Plans for the postmining land use are designed and certified by a registered professional engineer, in conformance with professional standards established to assure the stability, drainage and configuration necessary for the intended use of the site, appropriate professional, who shall ensure that the plans conform to applicable accepted standards for adequate land stability, drainage, vegetative cover and esthetic design appropriate for the postmining use of the site.

(6)] (3) ***

[(7)] (4) ***

[(8)] (5) ***

§ 88.231. Haul roads and access roads: general.

(a)	Haul roads and access roads shall be designed, constructed and maintained to [prevent to the maximum extent possible] CONTROL OR PREVENT erosion and [to prevent] contributions of sediment to streams or runoff outside the affected area; air and water pollution; damage to fish and wildlife or their habitat; flooding and damage to public or private property. Upon completion of the associated surface mining activities, the area disturbed by the road shall be restored in accordance with § 88.237 (relating to haul roads and access roads: restoration) unless retention of the road and its maintenance plan is approved as part of the postmining land use.

*        *        *        *        *

§ 88.237. Haul roads and access roads: restoration.

Unless the Department approves retention of a road as suitable for the approved postmining land use in accordance with § 88.221 (relating to postmining land use), [immediately] AS SOON AS PRACTICABLE after the road is no longer needed for the associated surface mining activities:

*        *        *        *        *

Subchapter D. ANTHRACITE REFUSE DISPOSAL: MINIMUM ENVIRONMENTAL PROTECTION PERFORMANCE STANDARDS

§ 88.283. Sealing of drilled holes: general requirements.

*        *        *        *        *

(e)	A solid barrier of undisturbed earth, 125 feet (38.1 METERS) in radius shall be maintained around all oil and gas wells, unless one of the following exists:

*        *        *        *        *

(2)	The Department approves, in writing, a lesser distance, provided that:

*        *        *        *        *

[(iii) The well operator agrees to such lesser distance.] (iii) THE WELL OPERATOR OF AN OPERATING OIL OR GAS WELL AGREES IN WRITING, TO THE LESSER DISTANCE.

*        *        *        *        *

§ 88.291. Hydrologic balance: general requirements.

(a)	Coal refuse disposal activities shall be planned and conducted to [prevent, to the maximum extent possible,] MINIMIZE disturbances to the prevailing hydrologic balance in [both] the permit and adjacent areas AND TO PREVENT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA.  THE DEPARTMENT MAY REQUIRE ADDITIONAL PREVENTIVE, REMEDIAL, OR MONITORING MEASURES TO ASSURE THAT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA IS PREVENTED.

*        *        *        *        *

§ 88.292.	Hydrologic balance: effluent standards.

(a)	Groups of effluent criteria. A person may not allow a discharge of water from an area disturbed by mining activities which exceeds the following groups of effluent criteria. The effluent limitations shall be applied under subsection (b).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 |  | greater than 6.0; less than 9.0 |  |
| alkalinity greater than acidity1 |  |  |  |

1 This parameter is applicable at all times.

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

*        *        *        *        *

§ 88.296. Hydrologic balance: sediment control measures.

Appropriate sediment control measures shall be designed, constructed and maintained to:

(1)	Prevent, to the [maximum] extent possible, additional contributions of sediment to streamflow or to runoff outside the affected area..

*        *        *        *        *

§ 88.334. Postdisposal land use.

(a)	All affected areas shall be restored in a timely manner to conditions that are capable of supporting the uses which they were capable of supporting before refuse disposal, or to higher or better uses achievable under criteria and procedures of this section and prior to the release of land from THE permit area in accordance with [this section (Bonds)] CHAPTER 86, SUBCHAPTER F (RELATING TO BONDING AND INSURANCE REQUIREMENTS.)

(b)	The predisposal use of land to which the postdisposal land use is compared shall be determined by the following:

*        *        *        *        *

[(3)	The postdisposal land use for land that has received improper management shall be judged on the basis of the predisposal use of surrounding lands that have received proper management.

(4)	The postdisposal land use of land which was changed within 5 years of the beginning of mining shall be judged on the basis of the historic use of the land as well as its use immediately preceding mining.]

(c)	Alternative land uses shall be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands and after determining that the following criteria are met:

(1)	The proposed postdisposal land use is compatible with adjacent land use and applicable land use policies, plans and programs and Federal, State and local law. A written statement of the views of the authorities with statutory responsibilities for land use policies and plans is submitted to the Department before refuse disposal activities begin. Any required approval, including any necessary zoning or other changes required for land use by local, State or Federal land management agencies, shall be obtained and remains valid throughout the [surface mining] COAL REFUSE DISPOSAL activities.

(2)	[The owner of the surface shall request in a notarized written statement that such alternative land use be approved.]  THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY ONE OR MORE OF THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(3)] (i)	Provision of any necessary public facilities is ensured as evidenced by letters of commitment from parties other than the person who conducts refuse disposal activities, as appropriate, to provide the public facilities in a manner compatible with the plans submitted under Subchapter A (relating to general provisions). The letters shall be submitted to the Department before [surface mining] COAL REFUSE DISPOSAL activities begin.

[(4)] (ii) ***

[(5)	Plans for the postdisposal land use are designed and certified by a registered professional engineer, in conformance with professional standards established to assure the stability, drainage and configuration necessary for the intended use of the site.

(6)] (3) ***

[(7)] (4) ***

[(8)] (5) ***

§ 88.335.	Haul roads and access roads: general.

(a)	Haul roads and access roads shall be designed, constructed and maintained to [prevent, to the maximum extent possible] CONTROL OR PREVENT erosion and [to prevent] contributions of sediment to streams or runoff outside the affected area; air and water pollution; damage to fish and wildlife or their habitat; and flooding and damage to public or private property. Upon completion of the associated surface mining activities, the area disturbed by the road shall be restored in accordance with § 88.341 (relating to haul roads and access roads: restoration) unless retention of the road and its maintenance plan is approved as part of the postmining land use.

*        *        *        *        *

§ 88.341.	Haul roads and access roads: restoration.

Unless the Department approves retention of a road as suitable for the approved postmining land use in accordance with § 88.334 (relating to postdisposal land use), [immediately] as soon as practicable after the road is no longer needed for the associated surface mining activities:

Subchapter F. ANTHRACITE UNDERGROUND MINES

§ 88.492. Minimum requirements for reclamation and operation plan.

*        *        *        *        *

(f)	Protection of [public] [PUBLICLY OWNED] PUBLIC parks and historic places.

(1)	For [a public park or historic place] PUBLICLY OWNED PARKS OR HISTORIC PLACES LISTED ON THE NATIONAL REGISTER OF HISTORIC PLACES that may be adversely affected by the proposed operations, each application shall describe the measures to be used to accomplish the following:

*        *        *        *        *

Subchapter G. ANTHRACITE SURFACE MINING ACTIVITIES AND ANTHRACITE BANK REMOVAL AND RECLAMATION ACTIVITIES: MINIMUM REQUIREMENTS FOR REMINING AREAS WITH POLLUTIONAL DISCHARGES

§ 88.509. Criteria and schedule for release of bonds on pollution abatement areas.

(a)	The Department will release up to [50] 60% of the amount of bond for the authorized pollution abatement area if the applicant demonstrates and the Department finds that:

*        *        *        *        *

(4)	The operator has not caused degradation of the baseline pollution load [for a minimum] AT ANY TIME DURING A PERIOD of 6 months prior to the submittal of the request for bond release under this subsection and until the bond release is approved as shown by all ground and surface water monitoring conducted by the permittee under § 88.506(1) (relating to operational requirements) or conducted by the Department.

*        *        *        *        *

(b)	The Department will release an additional amount [not to exceed 35% of the amount of the bond for the authorized pollution abatement area if the applicant demonstrates and the Department finds that:] OF BOND FOR THE AUTHORIZED POLLUTION ABATEMENT AREA BUT RETAINING AN AMOUNT SUFFICIENT TO COVER THE COST TO THE DEPARTMENT OF REESTABLISHING VEGETATION IF COMPLETED BY A THIRD PARTY IF THE OPERATOR DEMONSTRATES AND THE DEPARTMENT FINDS THAT:

*        *        *        *        *

(3)	The operator has complied with [either] ONE of the following:

*        *        *        *        *

(ii)	Achieved all of the following:

(A)	At a minimum has not caused degradation of the baseline pollution load as shown by all ground and surface water monitoring conducted by the operator or the Department:

(I)	For 12 months [from the date of initial bond release under subsection (a)] PRIOR TO THE DATE OF APPLICATION FOR BOND RELEASE AND UNTIL THE BOND RELEASE IS APPROVED UNDER SUBSECTION (b), if backfilling, final grading, drainage control, topsoiling and establishment of revegetation to achieve the standard of success for revegetation [set forth] in § 88.505(a)(5) have been completed[; or]

*        *        *        *        *

(c)	The Department will release the remaining portion of the amount of bond on the authorized pollution abatement area if the applicant demonstrates and the Department finds that:

*        *        *        *        *

(4)	The applicable liability period[, which shall be measured from the date of release of bond under subsection (b),] has expired under § 86.151 (relating to period of liability).

CHAPTER 89.

UNDERGROUND MINING OF COAL AND COAL PREPARATION FACILITIES

Subchapter B. OPERATIONS

INFORMATION REQUIREMENTS

§ 89.38. Archaeological and historical resources [and], public parks AND PUBLICLY OWNED PARKS.

*        *        *        *        *

(b)	For [a public park or historic place] PUBLICLY OWNED PARKS OR HISTORIC PLACES LISTED ON THE NATIONAL REGISTER OF HISTORIC PLACES that may be adversely affected by the proposed underground mining activities, the plan shall describe the measures to be used to accomplish one of the following:

*        *        *        *        *

PERFORMANCE STANDARDS

§ 89.52. Water quality standards, effluent limitations and best management practices.

*        *        *        *        *

(c)	Effluent limitations. A person may not allow a discharge of water from an area disturbed by underground mining activities, including areas disturbed by mineral preparation, processing or handling facilities which exceeds the following groups of effluent standards. The effluent limitations shall be applied under subsection (d).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 |  | greater than 6.0; less than 9.0 |  |
| alkalinity greater than acidity1 |  |  |  |

1 The parameter is applicable at all times.

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

*        *        *        *        *

§ 89.65. Protection of fish, wildlife and related environmental values.

(a)	The operator shall TO THE EXTENT POSSIBLE, using the best technology currently available, [prevent] MINIMIZE disturbances and adverse impacts of the activities on fish, wildlife and related environmental values, and achieve enhancement of the resources when practicable.

*        *        *        *        *

(d)	The operator shall TO THE EXTENT POSSIBLE, using the best technology currently available:

(1)	Locate and operate haul and access roads to [prevent] AVOID OR MINIMIZE impacts to fish and wildlife species or other species protected by State or Federal law.

(2)	[Prevent] AVOID disturbances to, ENHANCE WHERE PRACTICABLE, OR RESTORE, habitats of unusually high value for fish and wildlife.

*        *        *        *        *

§ 89.67. Support facilities.

(a)	Support facilities required for, or used incidentally to, the operation of the underground mine, including, but not limited to, mine buildings, coal loading facilities at or near the mine site, coal storage facilities, equipment storage facilities, fan buildings, hoist buildings, preparation plants, sheds, shops and other buildings, shall be [designed, constructed or reconstructed, and located to prevent or control erosion and sedimentation, water pollution and damage to public or private property. Support facilities shall be designed, constructed or reconstructed, maintained and used in a manner which, using the best technology currently available prevents:

(1)	Damage to fish, wildlife and related environmental values.

(2)	Additional contributions of suspended solids to streamflow or runoff outside the disturbed area. Contributions may not be in excess of limitations of State or Federal law.] LOCATED, MAINTAINED AND USED IN A MANNER THAT DOES THE FOLLOWING:

(1)	PREVENTS OR CONTROLS EROSION AND SILTATION, WATER POLLUTION AND DAMAGE TO PUBLIC OR PRIVATE PROPERTY.

(2)	TO THE EXTENT POSSIBLE USING THE BEST TECHNOLOGY CURRENTLY AVAILABLE:

(i)	MINIMIZES DAMAGE TO FISH, WILDLIFE AND RELATED ENVIRONMENTAL VALUES.

(ii)	MINIMIZES ADDITIONAL CONTRIBUTIONS OF SUSPENDED SOLIDS TO STREAMFLOW OR RUNOFF OUTSIDE THE PERMIT AREA. CONTRIBUTIONS MAY NOT BE IN EXCESS OF LIMITATIONS OF STATE OR FEDERAL LAW.

*        *        *        *        *

Subchapter C. RECLAMATION

PERFORMANCE STANDARDS

§ 89.82. Protection of fish, wildlife and related environmental values.

(a)	The operator shall, TO THE EXTENT POSSIBLE using the best technology currently available, [prevent] MINIMIZE disturbances and adverse impacts [of the reclamation activities] on fish, wildlife and related environmental values, and achieve enhancement of the resources where practicable.

(b)	The operator shall promptly report to the Department the presence in the permit area of threatened or endangered species under State or Federal laws of which that person becomes aware and which was not previously reported to the Department by that person. Upon notification, the Department will consult with the Game Commission or the Fish AND BOAT Commission and appropriate Federal fish and wildlife agencies and, after consultation, identify whether, and under what conditions, the operator may proceed.

(c)	The operator shall, TO THE EXTENT POSSIBLE using the best technology currently available:

*        *        *        *        *

(e)	Underground mining activities may not be conducted which are likely to jeopardize the continued existence of endangered or threatened species listed by the Secretary of the Interior, the Game Commission or the Fish AND BOAT Commission or which are likely to result in the destruction or adverse modification of designated critical habitats of these species in violation of the Endangered Species Act of 1973, the act of December 28, 1973 (Pub. L. No. 93-205) (87 Stat. 884).

§ 89.87. Regrading or stabilizing rills and gullies.

[(a)	When rills or gullies deeper than 9 inches form in areas that have been regraded and have had topsoil applied, the rills and gullies shall be filled, graded or otherwise stabilized and the areas reseeded or replanted according to § 89.86 (relating to revegetation). The Department may specify that rills or gullies of lesser size be stabilized, and the area reseeded or replanted, if the rills or gullies are disruptive to the approved postmining land use or may result in additional erosion and sedimentation.

(b)	The regrading or stabilization of rills and gullies to include the reseeding or replanting shall be completed no later than the first normal period for favorable planting as established within § 89.86, except rills or gullies contributing to impacts outside the permit area or those rills or gullies occurring after removal of erosion and sedimentation control structures shall be stabilized immediately.]

(a)	EXPOSED SURFACE AREAS SHALL BE PROTECTED AND STABILIZED TO EFFECTIVELY CONTROL EROSION AND AIR POLLUTION ATTENDANT TO EROSION.

(b)	RILLS AND GULLIES, WHICH FORM IN AREAS THAT HAVE BEEN REGRADED AND TOPSOILED AND WHICH DO ONE OF THE FOLLOWING SHALL BE FILLED, REGRADED OR OTHERWISE STABILIZED:

(1)	DISRUPT THE APPROVED POSTMINING LAND USE OR THE REESTABLISHMENT OF THE VEGETATIVE COVER.

(2)	CAUSE OR CONTRIBUTE TO A VIOLATION OF WATER QUALITY STANDARDS FOR RECEIVING STREAMS.

(c)	FOR AREAS LISTED IN SUBSECTION (B), THE TOPSOIL SHALL BE REPLACED, AND THE AREAS SHALL BE RESEEDED OR REPLANTED.

§ 89.88. Postmining land use.

*        *        *        *        *

(b)	The premining use of land to which the postmining land use is compared shall be those uses which the land previously supported, if the land had not been previously mined and had been properly managed.

*        *        *        *        *

[(2)	The postmining land use for land that has received improper management shall be judged on the basis of the premining use of surrounding lands that have received proper management.

(3)	If the premining use of the land was changed within 5 years of the beginning of mining, the comparison of postmining use to premining use shall include a comparison with the historic use of the land as well as its use immediately preceding mining.]

(c)	Alternative land uses may be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands, provided that:

*        *        *        *        *

(2)	THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(2)] (i) ***

[(3)	Plans for the postmining land use are designed under the general supervision of a registered professional engineer, or other appropriate professional, who will ensure that the plans conform to applicable accepted standards for adequate land stability, drainage, vegetative cover and esthetic design appropriate for the postmining use of the site.]

[(4)] (3) ***

[(5)] (4) ***

[(6)] (5) ***

[(7)] (6) ***

*        *        *        *        *

§ 89.90. Restoration of roads.

(a)	Unless the Department approves retention of a road as suitable for the approved postmining land use, [immediately] AS SOON AS PRACTICABLE after the road is no longer needed for operations, reclamation or monitoring:

*        *        *        *        *

(4)	Roadbeds shall be ripped[, plowed and] OR scarified.

*        *        *        *        *

CHAPTER 90. COAL REFUSE DISPOSAL

Subchapter A. GENERAL PERMIT AND APPLICATION REQUIREMENTS

FOR COAL REFUSE DISPOSAL

§ 90.1. Definitions.

The following words and terms, when used in this chapter, have the following meanings, unless the context clearly indicates otherwise:

*        *        *        *        *

Land use–Specific uses or management-related activities, rather than the vegetation or cover of the land. Land uses may be identified in combination when joint or seasonal uses occur. Changes of land use from one of the following categories to another shall be considered as a change to an alternative land use which is subject to approval by the Department.

*        *        *        *        *

(x)	UNMANAGED NATURAL HABITAT. IDLE LAND WHICH DOES NOT REQUIRE A SPECIFIC MANAGEMENT PLAN AFTER THE RECLAMATION AND REVEGETATION HAVE BEEN ACCOMPLISHED.

*        *        *        *        *

Subchapter C. MINIMUM OPERATION AND RECLAMATION PLAN INFORMATION REQUIRED IN APPLICATIONS FOR COAL REFUSE DISPOSAL

§ 90.40. Protection of [public] [PUBLICLY OWNED] PUBLIC parks and historic places.

(a)	For [public park or historic places] PUBLICLY OWNED PARKS OR HISTORIC PLACES LISTED ON THE NATIONAL REGISTER OF HISTORIC PLACES that may be adversely affected by the proposed coal refuse disposal activities, each application shall describe the measures to be used to accomplish the following:

(1)	[To prevent] PREVENT adverse impacts and meet the requirements of Chapter 86, Subchapter D (relating to areas unsuitable for mining).

(2)	[To minimize] MINIMIZE adverse impacts if valid existing rights exist or joint agency approval is to be obtained under Chapter 86, Subchapter D.

*        *        *        *        *

Subchapter D. PERFORMANCE [STANDARD] STANDARDS FOR COAL REFUSE DISPOSAL

§ 90.93. Casing and sealing of drilled holes and underground workings.

*        *        *        *        *

(d)	Gas and oil wells shall be sealed in accordance with [the requirements of] the [Gas Operations, Well-Drilling, Petroleum and Coal Mining Act (52 P. S. §§ 2101–2602).] OIL AND GAS ACT (58 P. S. §§ 601.101–601.605.)

(e)	A solid barrier of undisturbed earth, 125 feet (38.1 METERS) in radius, shall be maintained around all oil and gas wells, unless one of the following happens:

*        *        *        *        *

(2)	The Department approves, in writing, a lesser distance, provided:

*        *        *        *        *

[(iii)	The well operator agrees in writing to the lesser distance.]    (iii)    THE WELL OPERATOR OF AN OPERATING OIL OR GAS WELL AGREES, IN WRITING, TO THE LESSER DISTANCE.

§ 90.97. Topsoil: removal.

*        *        *        *        *

(c)	If topsoil is less than 12 inches (30.48 CENTIMETERS), a 12-inch (30.48 CENTIMETERS) layer of topsoil, subsoil and unconsolidated materials shall be removed, segregated, conserved and replaced as the final surface soil layer. If the topsoil and all unconsolidated material measures less than 12 inches (30.48 CENTIMETERS), all the topsoil, subsoil and unconsolidated material shall be removed and the mixture segregated and redistributed as the final surface soil layer.

*        *        *        *        *

(f)	When approved by the Department, in writing, other material may be substituted or used as a supplement to topsoil if the operator demonstrates that the resulting soil medium is equal or more suitable than topsoil for sustaining vegetation and soil productivity. In making this demonstration, the Department may require chemical and physical analyses of the substituted material and [subsoil] TOPSOIL. These analyses may include determinations of pH, net acidity or alkalinity, phosphorus, potassium, texture class, field site trials or greenhouse tests or other analyses as required by the Department.

§ 90.101. Hydrologic balance: general requirements.

(a)	Coal refuse disposal activities shall be planned and conducted to [prevent, to the maximum extent possible, changes] MINIMIZE DISTURBANCES to the prevailing hydrologic balance in the permit and adjacent areas AND TO PREVENT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA.    THE DEPARTMENT MAY REQUIRE ADDITIONAL PREVENTIVE, REMEDIAL, OR MONITORING MEASURES TO ASSURE THAT MATERIAL DAMAGE TO THE HYDROLOGIC BALANCE OUTSIDE THE PERMIT AREA IS PREVENTED.

*        *        *        *        *

§ 90.102. Hydrologic balance: water quality standards, effluent limitations and best management practices.

(a)	A person may not allow a discharge of water from an area disturbed by coal refuse disposal activities, including areas disturbed by mineral preparation, processing or handling facilities which exceeds the following groups of effluent criteria. The effluent imitations are to be applied under subsection (b).

Group A

|  | 30-day | Daily | Instantaneous |
| --- | --- | --- | --- |
| Parameter | Average | Maximum | Maximum |
| iron (total) | 3.0 mg/l | 6.0 mg/l | 7.0 mg/l |
| manganese (total) | 2.0 mg/l | 4.0 mg/l | 5.0 mg/l |
| suspended solids | 35 mg/l | 70 mg/l | 90 mg/l |
| pH1 |  | greater than 6.0; less than 9.0 |  |
| alkalinity greater than acidity1 |  |  |  |

1The parameter is applicable at all times.

Group B

| Parameter | Instantaneous Maximum |
| --- | --- |
| iron (total) | 7.0 mg/l |
| [manganese (total)] | [5.0 mg/l] |
| settleable solids | 0.5 ml/l |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

Group C

| Parameter | Instantaneous Maximum |
| --- | --- |
| pH | greater than 6.0; less than 9.0 |
| alkalinity greater than acidity |  |

*        *        *        *        *

§ 90.106. Hydrologic balance: erosion and sedimentation control.

(a)	Appropriate erosion and sediment control measures shall be designed, constructed and maintained using the best technology currently available to:

(1)	Prevent, to the [maximum] extent possible, contributions of sediment to stream flow or to runoff outside the affected area.

*        *        *        *        *

(3)	[Prevent] MINIMIZE erosion to the [maximum] extent possible.

*        *        *        *        *

§ 90.134. Haul roads and access roads: general.

(a)	Haul roads and access roads shall be designed, constructed and maintained to [minimize erosion and to prevent:] CONTROL OR PREVENT contributions of sediment to streams or runoff outside the affected area; flooding; air and water pollution; damage to fish and wildlife or their habitat; and damage to public or private property. To ensure environmental protection appropriate for their planned duration and use, including consideration of the type and size of equipment used, the design and construction or reconstruction of roads shall incorporate appropriate limits for grade, width, surface materials, surface drainage control, culvert placement and culvert size, in accordance with current, prudent engineering practices, and necessary design criteria established by the Department. Upon completion of the associated surface mining activities, the area disturbed by the road shall be restored in accordance with § 90.140 (relating to haul roads and access roads: restoration) unless retention of the road and its maintenance plan are approved as part of the postmining land use.

*        *        *        *        *

§ 90.140. Haul roads and access roads: restoration.

Unless the Department approves retention of a road as suitable for the approved postdisposal land use in accordance with § 90.166 (relating to postdisposal land use), [immediately] AS SOON AS PRACTICABLE after the road is no longer needed for operations, reclamation or monitoring:

*        *        *        *        *

(4)	Roadbeds shall be ripped[, plowed and] OR scarified.

*        *        *        *        *

§ 90.147.	Support facilities and utility installations.

(a)	Support facilities required for, or used incidentally to, the operation of the coal refuse disposal area, including, but not limited to, buildings, coal loading facilities at or near the coal refuse disposal site, coal storage facilities, equipment storage facilities, fan buildings, hoist buildings, preparation plants, sheds, shops and other buildings, shall be [designed, constructed or reconstructed, and located to prevent or control erosion and siltation, water pollution, and damage to public or private property. Support facilities shall be designed, constructed or reconstructed, maintained and used in a manner which prevents, using the best technology currently available:

(1)	Damage to fish, wildlife and related environmental values.

(2)	Additional contributions of suspended solids to streamflow or runoff outside the permit area. The contributions may not be in excess of limitations of State or Federal law.] LOCATED, MAINTAINED AND USED IN A MANNER THAT DOES THE FOLLOWING:

(1)	PREVENTS OR CONTROLS EROSION AND SILTATION, WATER POLLUTION AND DAMAGE TO PUBLIC OR PRIVATE PROPERTY.

(2)	TO THE EXTENT POSSIBLE USING THE BEST TECHNOLOGY CURRENTLY AVAILABLE:

(i)	MINIMIZES DAMAGE TO FISH, WILDLIFE AND RELATED ENVIRONMENTAL VALUES.

(ii)	MINIMIZES ADDITIONAL CONTRIBUTIONS OF SUSPENDED SOLIDS TO STREAMFLOW OR RUNOFF OUTSIDE THE PERMIT AREA. THESE CONTRIBUTIONS MAY NOT BE IN EXCESS OF LIMITATIONS OF STATE OR FEDERAL LAW.

*        *        *        *        *

§ 90.150. Protection of fish, wildlife and related environmental values.

(a)	A person conducting coal refuse disposal activities shall, TO THE EXTENT POSSIBLE using the best technology currently available:

(1)	[Prevent] MINIMIZE disturbances and adverse impacts of the activities on fish, wildlife and related environmental values, and achieve enhancement of the resources when practical.

(2)	[Prevent] LOCATE AND OPERATE HAUL AND ACCESS ROADS TO AVOID OR MINIMIZE impacts to fish and wildlife species or other species protected by State or Federal law.

(3)	[Prevent] AVOID disturbance to, ENHANCE WHERE PRACTICABLE, OR RESTORE, habitats of unusually high value for fish and wildlife.

*        *        *        *        *

(b)	A person who conducts coal refuse disposal activities shall promptly report to the Department the presence in the permit area of threatened or endangered species under State or Federal laws of which that person becomes aware and which was not previously reported to the Department by that person. Upon notification, the Department will consult with the Game Commission or the Fish AND BOAT Commission and appropriate Federal fish and wildlife agencies and, after consultation, will identify whether, and under what conditions, the operator may proceed.

*        *        *        *        *

(d)	Coal refuse disposal activities may not be conducted which are likely to jeopardize the continued existence of endangered or threatened species listed by the Secretary of the Interior, the Game Commission or the Fish AND BOAT Commission or which are likely to result in the destruction or adverse modification of designated critical habitats of the species in violation of the Endangered Species Act of 1973, act of December 28, 1973 (Pub. L. No. 93-205, 87 Stat. 884).

§ 90.166. Postdisposal land use.

(a)	Prior to the release of land from permit area in accordance with Chapter 86 [Subchapter E (relating to coal exploration)], SUBCHAPTER F (RELATING TO BONDING AND INSURANCE REQUIREMENTS), all affected areas shall be restored in a timely manner to conditions that are capable of supporting the uses which they were capable of supporting before any coal refuse disposal activities, or to higher or better uses achievable under criteria and procedures of this section.

(b)	The predisposal use of land to which the postdisposal land use is compared shall be determined by the following:

*        *        *        *        *

[(3)	The postdisposal land use for land that has received improper management shall be judged on the basis of the predisposal use of surrounding lands that have received proper management.

(4)	The postdisposal land use of land which was changed within 5 years of the beginning of coal refuse disposal activities shall be judged on the basis of the historic use of the land as well as its use immediately preceding disposal.]

(c)	Alternative land uses may be approved by the Department after consultation with the landowner or the land management agency having jurisdiction over the lands and after determining that the following criteria are met:

*        *        *        *        *

(2)	[The owner of the surface requests in a notarized written statement that such alternative land use be approved.] THE PROPOSED POSTMINING LAND USE IS REASONABLY LIKELY TO BE ACHIEVED WHICH MAY BE DEMONSTRATED BY ONE OR MORE OF THE FOLLOWING OR OTHER SIMILAR CRITERIA:

[(3)] (i) ***

[(4)] (ii) ***

[(5)] (iii) ***

[(6)	Plans for the postdisposal land use are designed and certified by a registered professional engineer, in conformance with professional standards established to assure the stability, drainage and configuration necessary for the intended use of the site.]

[(7)] (3) ***

[(8)] (4) ***

[(9)] (5) ***