COMMONWEALTH OF PENNSYLVANIA

Department of Environmental Protection

CONSOLIDATED GIS SYSTEM

INTRODUCTION

In order to effectively implement the many initiatives of ‘Reclaim PA’ which require the coordination of several program areas which are housed in several different    Bureaus within the Mineral Resources Management (MRM) Deputate,    a consolidated Geographic Information System (GIS) has been recommended for development.    The GIS is a tool to organize, analyze and view geographically-referenced data stored in electronic formats.    The storing of the geographic information and associated data in electronic formats will allow staff from many different programs located in any office with connections to the Department of Environmental Protection’s (DEP’s) Local Area Network (LAN) to access, view, analyze, update and produce hard copies of any of the information available in the GIS.    Data would also be available to those outside DEP via the Internet.    Users of the GIS may include DEP staff, staff from other government agencies, companies interested in remining, watershed associations and environmental groups interested in watershed rehabilitation and abandoned mined land (AML) reclamation, private citizens, consultants, educational institutions, and others.

The primary objectives of the GIS will be to build or develop data coverages that will:

- track all reclamation accomplishments occurring within the Commonwealth (including sites reclaimed by DEP, reclaimed by the active mining industry and sites reclaimed by others).

-include up-to-date and accurate information concerning all AML features located in the Commonwealth which would allow for comprehensive planning to focus AML and remining activities to the most appropriate sites as outlined in the Comprehensive Plan for Abandoned Mine Reclamation (CPAMR).

- incorporate an inventory of all known mine drainage discharges/impacts and incorporate cumulative hydrologic impact assessment concepts.

- facilitate implementation of an Areas Suitable for Remining Program.

- allow for the cooperation and coordination of all MRM program staff to improve everyday business decisions and procedures.

The MRM consolidated GIS development project is being coordinated with both the DEP GIS 1999 Project and the Bureau of Mining and Reclamation (BMR) Mine Permit Overlay and Deep Mine Map

digitization projects.    Both projects are being developed and completed independently of, but in coordination with, the MRM consolidated GIS development project.    The BMR GIS Project will be an integral part of the MRM consolidated GIS and will be extremely useful in the implementation of the ‘RECLAIM PA’ Program.

D R A F T

December 31, 1997

A consolidated GIS will serve as the ‘golden thread’ which will tie together all of the Bureaus and Programs within the MRM during and after the implementation of the ‘RECLAIM PA’ Program.    The easy access to all electronic information within the Deputate by all staff will facilitate improved planning, decision making, resource sharing, coordination, and accomplishment tracking.    The GIS will also allow unprecedented access to DEP information by groups and individuals outside of the DEP.    By allowing access to the GIS via the internet, countless agencies, groups, individuals, companies, etc. will able to participate at the grass roots level in the ‘RECLAIM PA’ Program.    Opportunities for Public/Private partnerships to undertake large scale environmental rehabilitation efforts such as restoration of large watersheds will be enhanced.    The ability to communicate and share up-to-date information (on a state-wide basis) will be possible as never before.

DISCUSSION

A detailed list was completed of information housed within the MRM Deputate which the committee recommended for inclusion in an MRM consolidated GIS.    The data all fit into one of three broad categories: Water Quality Data; Mineral Resource Features; and Investigation/Inspection Report Data.    A summary of these GIS data needs/desires is included in Appendix A.    A long list of external data needs was also identified.    GIS coverages of most of these data needs were determined to be already available from other sources such as the Pennsylvania Spatial Data Access (PASDA) web site or from other sources.

Since late September, a series of meetings have been held with the primary focus being the investigation of    how to proceed with converting the various data to electronic format, developing detailed background information for all of the MRM data recommended for inclusion in the GIS, estimating equipment, personnel, and training needs, and prioritizing the data conversion.    Conversion of BAMR’s AML Inventory Maps and NALIS database has been identified as the top priority for use by MRM staff in implementing ‘RECLAIM PA’, and a detailed implementation plan has been developed. The detailed implementation plan is included in Appendix B.

CONCLUSIONS

The committee determined that adequate equipment in the form of computers, software, large and small format scanners, plotters, and other hardware already exist within MRM or are planned for acquisition in the near future.

Training of staff will be necessary as the GIS system develops.    Training is available through the Office of Surface Mining’s (OSM’s) Technical Training Program and through PSU-ERRI.    Both provide hands-on training for ARC/VIEW and ARC/INFO, the two primary software packages which will utilized by higher level GIS users within MRM.    OSM also offers training on the use of global positioning systems (GPS).

Dedicated information/data management staff generally do not exist within most MRM offices.    These staff may include computer coordinators, system coordinators, network administrators, or GIS coordinators.    Several MRM offices are pursuing computer coordinators, however, GIS development and    maintenance has not been identified as a job duty or responsibility.

The consolidated GIS development project contains significant overlap with two other ‘RECLAIM PA’ initiatives.    Both the development of an Areas Suitable for Remining Program and the development of a Reclamation Clearinghouse have features or aspects which overlap or could be incorporated into the consolidated GIS development project.

The committee has no representation from the Oil and Gas Program, as of yet; however, an invitation was made recently to staff and management from Oil and Gas from the Southwest Regional Office of DEP to participate in the GIS development project.

Several Bureaus, including BAMR, are not included in the Bureau of Laboratories Sample Inventory System (SIS).    All water sample results from water samples collected by any staff from programs or Bureaus not on SIS, are stored on a Burroughs mainframe computer. This data is not stored in a format which would allow it to be linked to any GIS coverages.    All information in SIS can easily and directly be linked.

There are already two significant GIS development projects underway within the MRM Deputate.    BMR is in the process of developing a GIS coverage of all the mine permit boundary overlays and the WPA maps.    Permit information, presently housed on index cards, is being input into a database so that it can be linked with map coverages of the permit boundaries.    BAMR is presently pursuing an agreement with the Penn State University, Environmental Resources Research Institute (PSU-ERRI), to develop a state-wide GIS coverage of BAMR’s AML Inventory with links to the NALIS database.

The detailed implementation plan (Appendix B) developed for the conversion of BAMR’s AML Inventory to a state-wide GIS coverage can be used as a model for the development of other data coverages identified as GIS data needs/desires in Appendix A.

These efforts coupled with the DEP-wide GIS initiative known as the GIS 1999 Project show that a significant commitment to GIS has already been made.    The following recommendations by the Consolidated GIS Committee represent ideas on how to keep the GIS development activities moving forward, and how to address the long term needs of this new way of managing information within the MRM Deputate.

RECOMMENDATIONS

The following recommendations are being submitted by the committee for consideration in the development of a consolidated GIS for the MRM Deputate:

1)    The BMR GIS Project should continue to move forward.    Phases I-IV of the project have focused on developing a GIS coverage of BMR’s mine permit overlays and permit database.    Phase V is under early development and will deal with the conversion of underground mine maps from the Deputate’s various mine map repositories to electronic format.

2)    The BAMR GIS Project to develop a statewide GIS coverage of the AML inventory should also continue to be advanced according to the detailed implementation plan included in Appendix B.    The pending agreement between BAMR and PSU-ERRI will address the majority of implementation steps 1 through 4 of the 7 step plan.    The cost of this effort will be approximately $74,000 and will be paid for by BAMR with OSM Title IV funds.

3)    In order to implement steps 5 and 6 of the implementation plan for BAMR’s AML inventory (Appendix B), a small work team consisting of staff from BAMR, BMR, BDMO should be assembled to assess the present condition of the data contained in the AML Inventory, to evaluate the need to revise/modify/update the information, to develop alternatives for updating or adding data required to successfully implement the ‘RECLAIM PA’ Program, to coordinate the development of the Areas Suitable for Remining project and the Reclamation Clearinghouse, and to adapt the inventory to better suit the business needs of all of the programs within the MRM Deputate.    It is recommended that this team be established as soon as possible. (i.e. - Before the end of January 1998)

4)    MRM Deputate management should assemble a dedicated (full-time) GIS team to continue to prioritize additional desired GIS data coverages, develop detailed implementation plans, complete implementation of ‘RECLAIM PA’ GIS initiatives presently underway, and continue coordination with other ongoing GIS development projects (i.e.- DEP GIS 1999 Project, OSM Mine Map Digitization Project, BMR Mine Permit Inventory conversion Project, etc.).    A major task for this committee will be to evaluate how the developing GIS can improve job performance and service delivery and to demonstrate to management that the GIS development efforts will assist them in administering the many varied programs within the Deputate.    The committee discussed the idea of having an Information Management Coordinator at the Deputate level to coordinate the information management /GIS needs of all the MRM Bureaus and Programs.

5) Staff selected to serve on the above outlined team should be given a twelve to eighteen month special assignment or be permanently assigned to the GIS effort to insure success for this crucial project which will drive all of the MRM Bureaus and programs into the 21st century.    It is recommended that the team be limited to 4-6 staff with broad knowledge of the Deputate programs and GIS.    It is the committee’s opinion that the GIS development effort is doomed to be less than fully successful if the continued development is undertaken by staff with significant other responsibilities.    The committee recommends very strongly that the selected staff    not be expected to advance this initiative as a part-time pursuit.

6)    Staff from the Oil and Gas Program should be brought into the MRM consolidated GIS development project.

7)    BAMR’s (and maybe some other Bureaus’) water sampling analyses (as well as any other sampling being completed by DEP’s lab for mine drainage characterization) should be incorporated into the SIS.

8)    Plans should be developed to begin to educate and orient ALL MRM staff to the GIS initiative and the capabilities of GIS to improve all aspects of service delivery and job performance.    It is recommended that this be completed as soon as possible.    The GIS has enormous potential to change the way we all “do business”.    Education of both rank-and-file staff as well as management as to the uses and benefits of such a system must be undertaken or the system will not be supported.    It is our understanding that the DEP GIS Project Manager intends to establish a team to accomplish an orientation in the not too distant future.

9)    Any new or existing data sets being created, modified or updated within the MRM should begin to include data fields which would allow them to be spatially referenced. (i.e. - latitude/longitude, UTM’s, etc.)

COMMONWEALTH OF PENNSYLVANIA

Department of Environmental Protection

SUMMARY OF GIS DATA NEEDS/DESIRES

WATER QUALITY DATA

Various groups and individuals from outside and within the MRM Deputate collect water quality data. These include samples taken by inspection staff from the DMO’s, Oil & Gas, BAMR and BMR; sampling and monitoring information submitted with permit applications; self-monitoring data submitted by permitees; sample data compiled by watershed groups; sample data compiled during AMD Set Aside evaluations; CMRS project sample data; and other sample data.    Historical water sample data also exists such as that contained within the many Operation Scarlift watershed studies.    In addition, stream quality and sample data is compiled by other DEP Bureaus such as Water Quality Protection, by various federal agencies such as EPA , USGS, NRCS, and the Corps of Engineers, and by local organizations such as the Eastern and Western Pennsylvania Coalitions for Abandoned Mine Reclamation, watershed groups, and many others.

The data includes but is not limited to:	Format

1)	Sample Analyses contained within SIS	Electronic

2)	Permit files	Paper

3)	BMR and BAMR Project    & Planning Files	Paper

4)	Investigation/Inspection reports	Paper

5)	Scarlift data	Paper

6)	Post Mining Discharge Inventory	Electronic

7)	AMD Set Aside Site Evaluation Data	Paper/Electronic

8)	Watershed Group monitoring data	Various

MINERAL RESOURCE FEATURES

MINE PERMIT BOUNDARIES & WPA EXTENTS (Phase I-IV of the BMR GIS Project)

1)	Mylar Overlays for each Coal Seam on each 7.5’ USGS Quadrangle (350 Total Coal Bearing Quadrangles; 909 Total Overlays)

2)	Permit Inventory on Index Cards

AML INVENTORY (BAMR Entering into Agreement with PSU-ERRI to create GIS Coverage)

1)	Mylar Overlays

2)	NALIS Database

D R A F T

December 31, 1997

UNDERGROUND MINE MAPS AND FEATURES (Phase V of the BMR GIS Project)

1)	DMS Mine Map Repository (approx. 3,000+ blueprints)

2)	WPA MAPS

3)	McMurray & Wilkes-Barre Mine Map Repositories

4)	OSM Mine Map Repository

5)	Topo & Geo Mining Data

GEOLOGY & DRILLING INFORMATION

1)	Overburden analysis (OBA database)

2)	Exploration & permit drilling records

3)	BAMR drilling project records

4)	OSM & MSI drilling records

5)	Other geology related information

OIL & GAS WELL LOCATIONS

1)	Mylar overlays

2)	Information on FIX

AREAS UNSUITABLE FOR MINING

SOAP PERMIT AREAS

AREAS SUITABLE FOR REMINING

AFFECTED AREAS/RECLAIMED AREAS (within permitted mining sites)

1)	1” = 400’ paper maps for approximately 3200 permits (updated every six months for active operations)

AIR PHOTO DEVELOPMENT

BAMR PROJECT SITES

1)	OSM Tracking Report (OSM & AMD Set Aside Projects)

2)	BF Tracking Report

3)	AD/BD Projects

4)	OSM Emergency Projects

5)	CPAMR Project Sites

6)	BAMR Treatment Plant Sites

7)	No Cost Contract Project sites

DMO PROJECT SITES

1)	Rec in Lieu Projects

2)	No Cost Contracts

3)	Surety Reclamation Projects

4)	Remining Project Sites

5)	CMRS Project Sites

OTHER RECLAMATION PROJECT SITES

1)	NRCS PL 566 Project Sites

2)	EPA 319 Project Sites

3)	OSM Clean Streams Initiative Project Sites

4)	Active Operator Treatment Facilities

5)	Others

INVESTIGATION/INSPECTION INFORMATION

1)	BAMR Inquiries (approx. 1,000 per year)

2)	Subsidence Events/Claims (approx. 7,000 claims to date)

3)	Mining Complaints (approx. 2,500 per year)

COMMONWEALTH OF PENNSYLVANIA

Department of Environmental Protection

CONSOLIDATED GIS

IMPLEMENTATION PLAN

AML INVENTORY (NALIS Database and AML Map Overlays)

PRIORITY:    Incorporation of the AML Inventory into the MRM Consolidated GIS system should be the number one priority.    All of the other data to be included in the Consolidated GIS can be spatially related to either the USGS 7.5’ quadrangles or the problem area boundaries/features which are the primary geographic features depicted within the AML inventory.    Also, any new inventories such as BDMO’s remining inventory can be easily incorporated into the AML Inventory making more information available to more people at a single location.

Description:    Pennsylvania’s AML Inventory consists of the National Abandoned Lands Inventory System (NALIS) and AML inventory maps.    The inventory represents most of the abandoned coal mine related problems throughout the Commonwealth.    Specific problems have been grouped into units known as Problem Areas.    The Problem Area, or PA, is the primary inventory unit.    The inventory contains extensive information about each PA and each feature within the PA.

Why incorporate into MRM GIS?:  The AML inventory is the most comprehensive source of information about current AML problems within the Commonwealth.

Who would utilize the information?:    The AML inventory would be helpful to anyone interested in knowing the extent of any AML problems within a specific geographic area.

Where is the information presently housed?:    NALIS is a computerized database which is located on BAMR’s Central Office file server.    All BAMR offices have mylar copies of the AML inventory maps.    The Wilkes-Barre and Ebensburg District Offices also maintain a paper file which contains NALIS data and maps (in addition to other project development related information).

What is the format of the information?:    NALIS, which was originally a Burroughs main frame computer application, has recently been converted to a Microsoft Access database.    The basic AML inventory map is a transparent mylar overlay that corresponds to a specific USGS 7.5’ topographic map.    The NALIS database contains information which relates to each problem area in general (a 1:1 relationship), and information which relates to the individual abandoned mine land features (AMLF’s) depicted within each problem area boundary (a 1:many relationship).    The (PAD/IUF/Comments) information in NALIS relates to the entire problem area while the summary information relates to each AMLF depicted on the map overlays.

What is the accuracy/completeness of the information?:    The NALIS database and associated Map overlays were created in the early 1980’s for use by BAMR to plan and track PA’s abandoned mine reclamation program.    The inventory and data were set up to accommodate BAMR’s reporting requirements to OSM and to reflect as accurately as possible the AML problems which were eligible for funding under the OSM Title

D R A F T

December 31, 1997

IV program.    At that time, the focus of all AML efforts nationally was toward human health and safety

problems associated with abandoned coal mining operations.    Therefore, the inventory was setup to

primarily focus on these types of problems.    Over the first ten years of NALIS’ existence, OSM changed the criteria for inventorying various problems several different times.    More recently, OSM has changed the national AML focus to deal more with the various problems associated with mine drainage.    Therefore, the NALIS database and associated map overlays, particularly for the Bituminous District, do not accurately reflect the AML problems presently being focused upon (i.e. - mine drainage and water supply replacements).    Also, the information is not up-to-date and does not reflect the reclamation accomplishments of the active mining industry which have reclaimed many AML sites through remining.    Lastly, because of changes in the forms and information being submitted to OSM, the NALIS database is no longer able to support the business needs of BAMR in administering the OSM Title IV Program.

The Wilkes-Barre District Office has been continually updating NALIS since it was put on line in 1986.    While generally complete (up to date), the accuracy of NALIS information is widely variable.    PA’s that contain completed construction projects usually have very good corresponding data due to the benefit of accurate project mapping.    Undeveloped PA’s, however, probably only have “ball park” information.    Due to a lack of drafting support, the mylar overlays have not been updated since 1991.    The accuracy of the maps is also widely variable.    Large features, such as strip pits, that were delineated from aerial photography are usually well located.    Smaller features, like shafts, are often not precisely located.

The Ebensburg District and the Central Office staff historically shared responsibility to update and maintain the AML inventory for the Bituminous Coal District.    The last significant update effort occurred in 1989.    The Ebensburg Office continued to revise information in NALIS through early 1991.    Little maintenance or updating of the information in NALIS has been completed since then due to limited staff resources.

Is this information static or dynamic?:  The AML inventory is very dynamic, as known problems are reclaimed and as new problems are discovered.    Many of the features depicted in NALIS have been reclaimed and are not presently reflected as such.    Also, many, many previously unknown features have been investigated through BAMR’s inquiry investigation process and have not been added to the inventory.    Lastly, as the focus of the state’s reclamation efforts change, the AML inventory will need to be continually updated, maintained and modified to accurately plan and track the reclamation priorities and accomplishments of the Department.

What is the relative size or scope of the information to be incorporated into the GIS?:    The AML inventory contains 5485 Problem Areas (as of 19-Aug-1997).    The NALIS database contains data on approximately 30,900 individual abandoned mine land features.    The NALIS database presently has a file size of approximately 28 Megabytes.    There are 455 AML inventory maps that will need to be digitized, georeferenced and related to NALIS.    Of the 455 AML maps, only 350 are referenced in NALIS (meaning that a documented problem area exists on that map).

IMPLEMENTATION RECOMMENDATIONS

Step 1:    Convert AML Map Overlays to electronic form (Raster Images) & Georeference

Description:    The AML Overlays can be scanned using a large format scanner to obtain a raster image of the overlay.    Once scanned, the images can be registered and rectified (made compatible) for use with available electronic images of the USGS 7.5’ topographic maps.    Completion of this step would not be too costly and would allow the Deputate to make available the graphical information presently contained within BAMR’s AML Inventory to a large number of potential users including other Department employees, the general public, consultants, other government agency employees, real estate agents/appraisers, lending institutions, watershed groups, etc.

Resource Requirements:

Staffing Requirements: There are approximately 350 Quad overlays which have at least one problem area depicted, and approximately 100 additional overlays which fall within the coal field boundaries of Pennsylvania but which have no presently identified AML problems.    BAMR is presently entering into an agreement with Penn State University’s Environmental Resources Research Institute (PSU-ERRI) to complete this step.    The total cost of this effort is estimated at $8,500-$9,000, and will be paid for by BAMR.    Details of this project are outlined on the attached Task Summary.

Equipment Requirements:    It is estimated that 2 GB of storage space at a single location will be necessary to house the scanned, registered and rectified images.    This storage space is presently available within the Bureau.

The registering and rectifying process must be completed using the ARC/INFO software from ESRI.    One copy of this software is presently available in both District offices of BAMR as well as the Central Office of BAMR.    To utilize electronic images of the overlays, ARC/VIEW GIS software would be preferred for frequent users.    Four copies of this program presently are available within BAMR, and several more copies are planned for acquisition in the near future (early 1998).    The cost of this software is approximately $995/copy.    The Department has made available free copies of ESRI’s ARC Explorer program.    This is a very limited program that would allow casual users to view the information and make simple manipulations of the data.

In order to plot the raster images at large scale (i.e.-1:24,000), at least one copy of ESRI’s ARC/PRESS software will be required at each office location.

Recommended Timing:    In order to implement all of the initiatives outlined in the “RECLAIM PA“ framework, it is recommended that the conversion of BAMR’s AML overlays to electronic format begin as soon as possible.    Ideally this process should be complete by very early in 1998 so that the other related GIS and RECLAIM PA initiatives can move forward.    The proposed completion date for this phase is March 16, 1998.

Advantages/Drawbacks:	A- Can make AML Inventory widely accessible to DEP/Public

A- Would make AML overlays useable to GIS software for viewing

D- This step alone would not allow for a link to the NALIS data or other databases

D- The Overlays are not up-to-date and do not completely reflect the AML problems within Pennsylvania

A- Would allow remote users to view maps and identify problem areas of interest.    Users could then access NALIS through DEP Network to obtain NALIS Data

Availability:    This information could quickly and easily be made available to all DEP employees via the DEP Network (LAN).    This information could also be provided to the Pennsylvania Spatial Data Access (PASDA) for the purpose of providing access to this information to the general public.    The availability of this information through the Department’s GIS 1999 Project known as Pennsylvania Environmental Navigator Network (PENN) will be pursued as the GIS 1999 Project moves forward.

Training Requirements:    In the long run, a significant number of staff will need to be trained in the use of ARC/VIEW and probably ARC/INFO as well.    This training will facilitate the widespread use of the information and will provide the necessary foundation for the remaining implementation steps and the long range maintenance of the GIS system.

Step 2:    Convert Overlay Images from Raster to Vector Format

Resource Requirements:    BAMR is presently entering into an agreement with PSU-ERRI to complete this step.    The total cost of this effort is estimated at $65,000, and will be paid for by BAMR utilizing funds from the 1998 OSM Title IV Grant.    Details of this project are outlined on the attached Task Summary.

Staffing Requirements:    Staff from BAMR will be required to drop off and pick up the AML 	Inventory Overlays from PSU-ERRI.

Equipment Requirements:  No new equipment or software needs are anticipated for this step.

Recommended Timing: The proposed completion of this implementation step is December 31, 1998.    However, PSU-ERRI is going to do a small pilot with 4 quadrangles concurrently with the completion of step 1 outlined above, and the final completion and delivery date may be adjusted pending the outcome of the pilot project.

Advantages/Drawbacks: 	A - Completion of this step would allow for the linking of several MRM databases to the digitized overlays which would allow for the present and future storage, processing, retrieval, manipulation, and display of the spatially referenced data.    One of the primary advantages would be the ability for users to evaluate several different spatial data sets which would allow for the creation of new composite information which would be almost impossible to evaluate without GIS software.

Availability:    Same as that outlined for Step 1 above.

Training Requirements:    Some training of staff in order successfully link all of the database/ spreadsheet/ text documents to the vector images will probably be required.    PSU-ERRI will be providing some selected training to BAMR staff as part of the work agreement between BAMR and PSU-ERRI.

Step 3:    Construct Composite of all digitized (vector format) quadrangles to create a state-wide coverage of the AML Inventory

Resource Requirements:    BAMR is presently entering into an agreement with PSU-ERRI to complete this step.    The total cost of this effort is included in the cost for step 2 outlined above.    Details of this project are outlined on the attached Task Summary.

Step 4:    Establish Links between Various MRM Databases and Vector Map Images

Resource Requirements:    BAMR is presently entering into an agreement with PSU-ERRI to complete this step.    The total cost of this effort is included in the cost for step 2 outlined above.    Details of this project are outlined on the attached Task Summary.

Step 5:    Update AML Inventory

Resource Requirements: Yet to be determined.

Staffing Requirements: Yet to be determined.

Equipment Requirements: Yet to be determined.

Recommended Timing: Yet to be determined.

Advantages/Drawbacks: Yet to be determined.

Availability: Yet to be determined.

Training Requirements: Yet to be determined.

Step 6:    Enhance NALIS and other existing databases in order to meet business needs of MRM Programs, to facilitate implementation of ‘RECLAIM PA’ Program, to be compatible with the Department’s GIS 1999 Project goals, and to meet needs of GIS users both within and outside of DEP.

Resource Requirements: Yet to be determined.

Staffing Requirements: Yet to be determined.

Equipment Requirements: Yet to be determined.

Recommended Timing: Yet to be determined.

Advantages/Drawbacks: Yet to be determined.

Availability: Yet to be determined.

Training Requirements: Yet to be determined.

Step 7:    Establish Procedures and Allocate Resources to Maintain and Update both the Data and the GIS System for the foreseeable future

Resource Requirements: Yet to be determined.

Staffing Requirements:    This will probably require at least one person whose full-time duties are to serve as a GIS coordinator in each office within the MRM Deputate.

Equipment Requirements: Yet to be determined.

Recommended Timing: Yet to be determined.

Advantages/Drawbacks: Yet to be determined.

Availability: Yet to be determined.

Training Requirements: Yet to be determined.

COMMONWEALTH OF PENNSYLVANIA

Department of Environmental Protection

ATTACHMENT

TASK SUMMARY

Task  36 :    Convert BAMR’s AML Inventory to a GIS

# Purpose

To develop a Statewide GIS of BAMR’s AML Inventory that will serve as a core component to access various types of data including databases, photographs, scanned file documents, etc.

Overview

Description:    Pennsylvania’s AML Inventory consists of the National Abandoned Lands Inventory System (NALIS) and AML inventory map overlays.    The inventory represents most of the abandoned coal mine related problems throughout the Commonwealth.    Specific problems (features) have been grouped into units known as Problem Areas.    The Problem Area, or PA, is the primary inventory unit.    The inventory contains extensive information about each PA as a whole and specific information on each feature within the PA’s.

Where is the information presently housed?:    NALIS is a computerized database which is located on BAMR’s Central Office file server.    All BAMR offices have mylar copies of the AML inventory maps.    The original mylars for the bituminous district are housed in Ebensburg and the originals for the anthracite district are housed in Wilkes-Barre.

What is the format of the information?:    NALIS is a Microsoft Access (Ver. 2.0) database.    The AML inventory maps are transparent mylar overlays that correspond to a specific USGS 7.5’ topographic map.    Each overlay is reference marked to the corners of the quad.    The NALIS database contains information which relates to each problem area on the map (a 1:1 relationship), and information which relates to the individual abandoned mine land features (AMLF’s) depicted within each problem area (a 1:many relationship).    Three tables (PAD/IUF/Comments) in NALIS relate to the entire problem area while the Summary table relates to each individual feature depicted on the map overlays.

Is this information static or dynamic?:  The AML inventory is very dynamic as known problems are reclaimed and as new problems are discovered.    Many of the features depicted in NALIS are routinely reclaimed and must be updated as such.    Also, previously unknown features are frequently investigated through BAMR’s inquiry investigation process and must be added to the inventory.    Lastly, as the focus of the state's reclamation efforts changes, the AML inventory will need to be continually updated, maintained and modified to accurately plan and track the reclamation priorities and accomplishments of the Department.    The system being developed must be dynamic and easily updated by existing staff.

What is the relative size or scope of the information to be incorporated into the GIS?:    The AML inventory contains 5485 Problem Areas (as of 19-Aug-1997).    The NALIS database contains data on approximately 30,900 individual abandoned mine land features.    The NALIS database presently has a file size of approximately 28 Megabytes.    There are 455 AML inventory maps that will need to be scanned, georeferenced, digitized, and related to NALIS.    Of the 455 AML maps, only 350 are referenced in NALIS (meaning that a documented problem area exists on that map).

D R A F T

December 31, 1997

Work Items

Phase 1:	Scan and Georeference AML Inventory Maps

Details:

- The scanning resolution should be 400 dpi or higher.

- If possible the scanned files should be named similar to the USGS naming convention for 7.5’ quadrangle maps (o40079a5).    The names should be eight characters or less plus the file extension.

- The inventory maps will be delivered and picked up by BAMR staff.    The maps can be delivered as either one large group or several smaller sets.

Deliverables:

- A georeferenced TIFF file for each overlay.    The TIFF format should be compatible for viewing with ARC Explorer, ARC/VIEW, and ARC/INFO.    The completed set of files is to be delivered on CD-ROM(s).

- Three copies of the CD-ROM(s) are to be provided.

Due Date:

- All deliverables from Phase 1 are due by March 16, 1998

Phase 2:	Digitize and develop a Pilot GIS based on four AML Overlays (quadrangles)

Details:

- The Pilot will be based on an area covering four quadrangles.    The quadrangles to be used are the Wallaceton, Philipsburg, Houtzdale, and Sandy Ridge USGS 7.5’ quadrangles.

- The individual quads should be seamed together into a single coverage

- All items on the maps are to be digitized, labeled, and given attributes so that links can be established to the NALIS database.    Symbol sets should be built to allow maps to be generated that provide a level of symbolic information comparable to the original overlays.

- Sufficient functionality should be built into the system to demonstrate how average/novice users can access information.    Additional functionality should include updating maps, printing maps of an individual PA, printing 7.5’ quad based maps at full and reduced scale, linking to other data sources, querying the data and generating necessary reports.

Deliverables:

- All project, coverage and data files necessary for the pilot system to operate.    The completed set of files is to be delivered on CD-ROM(s).

- Three copies of the CD-ROM(s) are to be provided.

Due Date:

- Phase 2 should run concurrent with Phase 1.    All deliverables from Phase 2 are due by March 2, 1998

Phase 3:	Digitize remaining AML Overlays

Details:

- The remaining AML Overlays will be digitized to the standards established through the pilot.

- Develop a short (1-2 day duration) training session to introduce and orient staff to the new GIS and to demonstrate its functionality.

Deliverables:

- All project, coverage, and data files necessary for the system to operate.    The completed set of files is to be delivered on CD-ROM(s).

- Quarterly progress updates of the developing AML coverage are to be provided on the first of April, July, and October, 1998.

- Three copies of the final system and the Quarterly progress updates are to be provided on CD-ROM(s).

Due Date:

- All deliverables from Phase 3 are due by December 31, 1998.    Quarterly progress updates are due as described above.

- It is anticipated that staff training would be provided sometime during the fourth quarter of 1998.