CHAPTER 102. EROSION CONTROL 

GENERAL PROVISIONS 

Sec. 

Definitions. 

102.2. Scope AND PURPOSE. 

102.3. [Purpose] RESERVED. 

General. 

Erosion and [sedimentation] SEDIMENT  control plan. 

EROSION AND [SEDIMENTATION] SEDIMENT CONTROL 

[MEASURES AND FACILITIES] BEST MANAGEMENT PRACTICES

102.11. General requirements. 

102.12. [Control measures] BEST MANAGEMENT PRACTICES. 

102.13. [Control facilities.] RESERVED. 

[RESTORATION] FINAL STABILIZATION

102.21.[Applicability.] RESERVED. 

102.22. PERMANENT Stabilization

102.23. [Interim control measures.] RESERVED. 

102.24. [Final measures.] REMOVAL OF BEST MANAGEMENT PRACTICES. 

PERMITS [AND PLANS]

102.31. Permit requirements. 

102.32. Application for permit. 

RESPONSIBILITIES OF LOCAL GOVERNING BODIES 

102.41. Administration by local governing bodies. 

102.42. Notification of application for [building] permits. 

102.43. Withholding [building] permits. 

IMPLEMENTATION 

102.51. [Effective dates] (RESERVED). 



GENERAL PROVISIONS 

§ 102.1. Definitions. 

The following words and terms, when used in this chapter, have the following meanings, unless the context clearly indicates otherwise:

 Accelerated erosion–The removal of the surface of the land through the combined action of [man's] HUMAN activities and the natural processes, at a rate greater than would occur because of the natural process alone. 

AGRICULTURAL PLOWING AND TILLING-EARTH DISTURBANCE ACTIVITY FOR THE PRODUCTION OF AGRICULTURE CROPS.

BEST MANAGEMENT PRACTICES (BMPs)- ACTIVITIES, FACILITIES, MEASURES, OR PROCEDURES USED TO  PREVENT  SEDIMENT POLLUTION TO WATERS OF THE COMMONWEALTH,  AND TO PROTECT IN-STREAM AQUATIC USES.

 COLLECTOR- A FACILITY, INCLUDING A CHANNEL, DIKE, OR  OTHER CONVEYANCE, CONSTRUCTED DOWNSLOPE OF  AN EARTH DISTURBANCE ACTIVITY FOR THE PURPOSE OF COLLECTING RUNOFF  FROM  EXISTING OR PROPOSED DISTURBED AREA

Conservation plan–A plan that identifies [conservation practices] BEST MANAGEMENT PRACTICES  FOR AGRICULTURAL PLOWING AND TILLING ACTIVITIES. [and, at the very least, includes an erosion and sedimentation control  plan.]

Department–The  Department of Environmental Protection of the Commonwealth. 

Diversion [terrace]–A channel,  TERRACE, or dike constructed up-slope of [a project] AN  EARTH DISTURBANCE ACTIVITY for the purpose of diverting [storm water] RUNOFF away from [the unprotected slope] AN EXISTING OR PROPOSED DISTURBED AREA. 

DISTURBED AREA-LAND SUBJECT TO AN EARTH DISTURBANCE ACTIVITY.

[Earthmoving] EARTH DISTURBANCE activity–A construction or other HUMAN activity  which disturbs the surface of the land, including, but not limited to, GRUBBING, excavations, embankments, land development, [subdivision development,] AGRICULTURAL PLOWING AND TILLING, mineral extraction, and the moving, depositing, STOCKPILING, or storing of soil, rock or earth MATERIALS. 

[Embankment or fill–A deposit of soil, rock or other material placed by man.]

Erosion–The natural process by which the surface of the land is worn away by [the action of] water, wind or chemical action. 

Erosion and [Sedimentation] SEDIMENT Control Plan–A  SITE-SPECIFIC plan [which is designed] IDENTIFYING BEST MANAGEMENT PRACTICES OR OTHER SITE-SPECIFIC PRACTICES  to minimize accelerated erosion and [sedimentation] TO PREVENT POLLUTION FROM EARTH DISTURBANCE ACTIVITIES. 

[Excavation–A cavity formed by digging, quarrying, uncovering, displacing or relocating soil or rock.]

[Interceptor channel–A channel or dike constructed across a slope for the purpose of intercepting stormwater, reducing the velocity of flow and diverting it to outlets where it may be disposed.]

INTERIM  STABILIZATION -TEMPORARY COVERING OF A DISTURBED AREA , WHILE IMPLEMENTING AND MAINTAINING OTHER BEST MANAGEMENT PRACTICES.

[Land developer–A person who is engaged in land development as the principal rather than an agent or contractor.] 

Land development–[The] ANY [constructing, installing, placing, planting or building of surface structures] CONSTRUCTION, INSTALLATION, DEMOLITION, OR EARTH DISTURBANCE ACTIVITIES ASSOCIATED WITH LAND USE CHANGES INCLUDING BUT NOT LIMITED TO utility lines,[shopping centers and malls] COMMERCIAL AND RESIDENTIAL DEVELOPMENT, golf course DEVELOPMENT, [apartment complexes, schools,] roads, highways and parking areas or other similar [activity] ACTIVITIES. 

Municipality–SHALL BE CONSTRUED TO INCLUDE ANY [A] county, city, borough, town, township, school district, institution or authority created by any one or more of the foregoing. [For purposes of this definition, town shall include an incorporated town. ]

Person–SHALL BE CONSTRUED TO INCLUDE ANY [A] natural person, partnership, association or corporation[.] OR ANY AGENCY, INSTRUMENTALITY, OR ENTITY OF FEDERAL OR STATE GOVERNMENT.

PROJECT-THE ENTIRE PROPOSED ACTIVITY REGARDLESS OF THE  ACREAGE OF THE EARTH DISTURBANCE.

ROAD  MAINTENANCE- AN INSIGNIFICANT  EARTH DISTURBANCE ACTIVITY REQUIRED TO ROUTINELY REPAIR  AN UNPAVED ROAD SURFACE,  CONDUCT SHOULDER GRADING, CUTTING OR STABILIZATION,  CLEAN OR RESHAPE DRAINAGE FACILITIES,  REPAIR EMERGENCY DAMAGE,  AND OTHER RELATED MINOR EARTHMOVING ACTIVITIES.

Sediment–Soils or other [surficial] materials transported by surface water as a product of erosion. 

[Sedimentation–The process by which sediment is deposited on stream bottoms.] 

SEDIMENT POLLUTION- THE PLACEMENT, DISCHARGE OR OTHER INTRODUCTION OF MATERIALS FROM AN EARTH DISTURBANCE ACTIVITY INTO THE WATERS OF THE COMMONWEALTH .

Stabilization–The proper placing, grading, COMPACTING, CONSTRUCTING, REINFORCING, LINING, OR [and] covering of soil, rock or earth to insure their resistance to erosion, sliding or other movement. 

[Subdivision–The division or redivision of a lot, tract or parcel of land by a means into two or more lots, tracts, parcels or other division of land, including changes in existing lot lines for the purpose, whether immediate or future, of lease, transfer of ownership or building or lot development.]

TIMBER HARVESTING ACTIVITIES-EARTH DISTURBANCE ACTIVITIES RESULTING FROM THE CONSTRUCTION OF SKID TRAILS, LOGGING ROADS,  LANDING AREAS, AND OTHER RELATED LOGGING PRACTICES. 

§ 102.2. Scope[.] AND  PURPOSE. 

This chapter imposes requirements [on earthmoving activities which create accelerated erosion or a danger of accelerated erosion and which require planning and implementation of effective soil conservation measures] FOR PLANNING, IMPLEMENTING AND MAINTAINING BEST MANAGEMENT PRACTICES FOR EARTH DISTURBANCE ACTIVITIES TO MINIMIZE ACCELERATED EROSION AND TO PREVENT SEDIMENT POLLUTION. 

 [§ 102.3. Purpose. ] RESERVED.

[This chapter controls accelerated erosion and the resulting sedimentation of waters of this Commonwealth thereby preventing the pollution of these waters from sediment and from fertilizers, pesticides and other polluting substances carried by sediment. ]





§ 102.4. General. 

[Earthmoving] EARTH DISTURBANCE activities within this Commonwealth shall be conducted in such a way as to [prevent] MINIMIZE accelerated erosion and [the resulting sedimentation.] TO PREVENT SEDIMENT POLLUTION.  To accomplish this, [except as provided in subsection (b)] a landowner, person or municipality engaged in [earthmoving] EARTH DISTURBANCE activities shall develop, implement and maintain erosion and [sedimentation] SEDIMENT control [measures] BEST MANAGEMENT PRACTICES AND ANY OTHER CONTROLS REQUIRED [which effectively  minimize] TO MINIMIZE accelerated erosion and [sedimentation] TO PREVENT  SEDIMENT POLLUTION.  [These erosion and sedimentation measures shall be set forth  i n a  plan as set forth in § 102.5 (relating to erosion and  sedimentation control  plan) and be available at all times at the site of the activity.  The Department, or  its designee may, at its discretion, require this plan to be filed with the Department or its designee. ] 

IN ADDITION TO THE IMPLEMENTATION AND MAINTENANCE OF EROSION AND SEDIMENT CONTROL BEST MANAGEMENT PRACTICES REQUIRED IN § 102.4 (a), THE FOLLOWING EARTH DISTURBANCE ACTIVITIES REQUIRE THE DEVELOPMENT OF A WRITTEN EROSION AND SEDIMENT CONTROL PLAN DETAILING BEST MANAGEMENT PRACTICES AS SET FORTH IN § 102.5:

PROJECTS OF 5,000 OR  SQUARE FEET  ( 464 SQUARE  METERS) OR MORE OF TOTAL EARTH DISTURBANCE ;  

PROJECTS REQUIRING AN EROSION AND SEDIMENT CONTROL PLAN PURSUANT TO OTHER DEPARTMENT REGULATIONS;

PROJECTS WHICH THE DEPARTMENT DETERMINES MAY ADVERSELY IMPACT WATERS OF THE  COMMONWEALTH; AND

PROJECTS WHICH MAY DISCHARGE TO A WATER BODY CLASSIFIED AS HIGH QUALITY OR EXCEPTIONAL VALUE  IN CHAPTER 93 OF THIS TITLE. 

(c)  THE WRITTEN PLAN DESCRIBED IN § 102.4 (b) , SHALL BE AVAILABLE AT PROJECT SITE FOR REVIEW AND INSPECTION DURING ALL STAGES OF THE EARTH DISTURBANCE ACTIVTY.  THE DEPARTMENT OR ITS DESIGNEE MAY REQUEST THAT THE PLAN BE SUBMITTED FOR REVIEW AND APPROVAL.

[(b)] (d) In the case of agricultural  plowing and tilling, the landowner, OR LAND OCCUPIER  shall be JOINTLY AND SEVERABLY  responsible for developing, [and] implementing, AND MAINTAINING  the [erosion and sedimentation control plan.] CONSERVATION PLAN.  [The landowner may delegate  his obligation to implement  the plan or parts thereof, by written agreement, to a tenant or lessee. If an agreement exists, the tenant or lessee shall be responsible for implementing those provisions delegated under the agreement.]

§ 102.5. Erosion and [sedimentation] SEDIMENT control plan. 

(a) The erosion and [sedimentation] SEDIMENT control plan shall be prepared by a person trained and experienced in erosion and [sedimentation] SEDIMENT control methods and techniques. 

(b) The erosion and [sedimentation] SEDIMENT control plan shall be designed to [prevent] MINIMIZE accelerated erosion and [sedimentation]TO PREVENT SEDIMENT POLLUTION.  [and]  THE PLAN shall consider all factors which HAVE THE POTENTIAL TO contribute to erosion and [sedimentation] SEDIMENT POLLUTION, including, but not limited to, the following: 

    (1) The EXISTING topographic features of the project [area], INCLUDING CRITICAL AND IMMEDIATE SURROUNDING AREAS,  AS NEEDED TO                                 EVALUATE THE EFFECTIVENESS OF THE PLAN. 

(2) The types, depth, slope, [and areal extent] LOCATIONS AND LIMITATIONS of the soils. 

(3) The proposed alteration to the area.

(4) The amount of runoff from the project area and the upstream watershed area. 

(5) The staging of [earthmoving] BEST MANAGEMENT PRACTICE INSTALLATION AND REMOVAL, PRIOR  TO, DURING , AND AFTER  EARTH     DISTURBANCE activities. 

[(6) Temporary control measures and facilities for use during earth moving. 

Permanent control measures and facilities for long term protection.] 

(6)   THE TEMPORARY AND PERMANENT BEST MANAGEMENT PRACTICES AND OTHER NECESSARY MEASURES REQUIRED FOR  THE PREVENTION OF SEDIMENT POLLUTION, THE NATURE OF THE EARTH DISTURBANCE ACTIVITY, AND THE METHOD(S) FOR STABILIZATION OF THE PROJECT.

[(8)] (7) A maintenance program for the [control facilities] BEST MANAGEMENT PRACTICES.[including] THE  MAINTENANCE  PROGRAM SHALL INCLUDE  REGULAR INSPECTION, REPAIR , AND OPERATION [disposal of materials removed from the control facilities or project area.] OF THE BEST MANAGEMENT PRACTICES. 

(8)  SPECIAL PLANNING CONSIDERATIONS ARE REQUIRED FOR PROJECTS CONTAINING SEVERE  LAND USE LIMITATIONS, OR FOR PROJECTS DISCHARGING TO WATERBODIES CLASSIFIED AS HIGH QUALITY OR EXCEPTIONAL VALUE  IN CHAPTER 93 OF THIS TITLE.

IN THE CASE  OF AN AGRICULTURAL PLOWING OR TILLING  ACTIVITY,  A CONSERVATION PLAN WHICH  ADDRESSES ITEMS IN §102.5(c) AND INCLUDES LAND USE, TILLAGE SYSTEMS, SCHEDULES, AND COST  EFFECTIVE AND TECHNICALLY PRACTICAL CONSERVATION TREATMENT MEASURES WILL BE CONSIDERED BY THE DEPARTMENT.

EROSION AND [SEDIMENTATION] SEDIMENT CONTROL [MEASURES

AND FACILITIES] BEST MANAGEMENT PRACTICES

§ 102.11. General requirements. 

The erosion and [sedimentation] SEDIMENT  control [facilities] BEST MANAGEMENT PRACTICES set forth in §§ 102.12 [and 102.13 (relating to control measures; and control facilities)] (RELATING TO BEST MANGEMENT PRACTICES) shall be appropriately incorporated into ALL [earthmoving] EARTH DISTURBANCE activities unless the designer of the erosion and [sedimentation] SEDIMENT control plan shows that alteration of these [measures and facilities] BEST MANAGEMENT PRACTICES or inclusion of other [measures and facilities] SOUND CONSERVATION AND ENGINEERING PRACTICES shall  [prevent] MININMIZE  accelerated erosion and [sedimentation] PREVENT SEDIMENT POLLUTION.   THE  ALTERATION OF  BEST MANAGEMENT PRACTICES OR THE INCLUSION OF OTHER SOUND CONSERVATION AND ENGINEERING PRACTICES MUST BE CONSISTENT WITH AND NO LESS STRINGENT THAN THOSE CONTAINED IN THIS CHAPTER. 







§ 102.12. [Control measures.] BEST MANAGEMENT PRACTICES. 

BEST MANAGEMENT PRACTICES INCLUDE, BUT ARE NOT LIMITED TO THE FOLLLOWING: 

[(a)] (1) Limiting exposed areas. [Earthmoving] EARTH DISTURBANCE activities shall be planned AND CONDUCTED in such a manner as to minimize the [areal] extent of disturbed land. 

[(b)] (2) Surface water diversion. Surface water shall be diverted away from [the project area] ALL DISTURBED AREAS .  TEMPORARY BEST MANAGEMENT PRACTICES THAT  DIVERT OR CARRY  SURFACE WATER SHALL BE DESIGNED TO HAVE SUFFICIENT CAPACITY TO CONVEY 1.6 CUBIC FEET PER SECOND (CFS) PER ACRE ( 0.1 CUBIC METERS PER SECOND PER HECTARE) OF TRIBUTARY DRAINAGE, OR THE PEAK DISCHARGE FROM A TWO-YEAR/24-HOUR FREQUENCY STORM EVENT.  THE CAPACITY SHALL BE 2.75 CFS PER ACRE  (0.2 CUBIC METERS PER SECOND PER HECTARE)  OF TRIBUTARY DRAINAGE, OR THE PEAK DISCHARGE FROM A TEN-YEAR/24 HOUR FREQUENCY STORM EVENT FOR PERMANENT BEST MANAGEMENT PRACTICES THAT DIVERT OR CARRY SURFACE WATER.   DIVERSIONS SHALL BE INSTALLED AND STABILIZED PRIOR TO INITIATION OF  OTHER PROJECT SITE EARTH DISTURBANCE  ACTIVITIES.

[(c)] (3) Velocity control. [Permanent facilities for the conveyance of] TEMPORARY AND PERMANENT DIVERSIONS AND CHANNELS WHICH CONVEY  SURFACE  water around, through or from the project area shall be designed [or contain facilities] to limit the velocity of flow [in the facilities] IN THE DIVERSION OR CHANNEL to [less than 1.5] 2.0 feet per second (0.6 CUBIC METERS PER SECOND).  THE DIVERSION OR CHANNEL SHALL 

BE PROVIDED WITH A SUITABLE EROSION RESISTANT LINING, UNLESS THE  PLAN DESIGNER DEMONSTRATES THAT VELOCITY WILL BE CONTROLLED TO 2.0 FEET PER SECOND (0.6CUBIC METERS PER SECOND) OR LESS. 

[(d)] (4) PERMANENT Stabilization. [Slopes, channels, ditches or a] ALL disturbed [area] AREAS shall be IMMEDIATELY stabilized [as soon as possible] after the final grade or final [earth moving has been] EARTH DISTURBANCE ACTIVITES ARE completed. 

[(e)] (5) Interim stabilization. If it is not possible to permanently stabilize a disturbed area immediately after the final earth [moving] DISTURBANCE ACTIVITY  has been completed  or  where the activity ceases for more than 20 days,  interim stabilization [measures] AND OTHER BEST MANAGEMENT PRACTICES shall be implemented [promptly] IMMEDIATELY. 

[(f)] (6) Collection of runoff. Runoff from a project area shall be collected and diverted to [facilities] APPROPRIATE BEST MANAGEMENT PRACTICES for removal of sediment. 

[(g)] (7) Solids separation. Runoff from a project area may not be discharged into the waters of this Commonwealth without means to[ prevent sedimentation.] MINIMIZE ACCELERATED EROSION AND TO PREVENT SEDIMENT POLLUTION.

 [§ 102.13. Control facilities.] 

[(a)] (8) [Diversion terraces] DIVERSIONS. [The following applies to diversion terraces: ]

[(1)] (i) TEMPORARY OR PERMANENT  [Diversion terraces] DIVERSIONS shall be [constructed up-grade] INSTALLED UPSLOPE  of a [project] DISTURBED area PRIOR TO INITIATION OF EARTH DISTURBANCE ACTIVITIES to convey runoff around the [project] DISTURBED area. [For temporary diversion, the channel shall have a capacity to convey 1.6 cubic feet per second per acre of land tributary to it. For permanent diversion, the channel shall have a capacity to convey 2.75 cubic feet per second per acre of land tributary to it.] 

[(2) Diversion terraces shall be grassed or lined with erosion resistant material to prevent accelerated erosion within the channel . 

(3) Outlet structures shall be designed to maintain a discharge velocity of less than three feet per second and shall be stabilized before use.] 

[(b) Interceptor channels. The following applies to interceptor channels: 

(1) Interceptor channels may be used within a project area to reduce the velocity of flow and thus prevent accelerated erosion. 

(2) Water collected by interceptor channels shall be conveyed to sedimentation basins or to vegetated areas but not directly to streams. 

(3) Outlets to vegetated areas shall be designed to maintain an outlet velocity of less than three feet per second. ]

(ii)  [Channels of conveyance. Channels used to convey water through a project area shall be designed to have a velocity of less than 1 1/2 feet per second. If this is not possible, the channel shall be grassed or lined with erosion resistant material. ]   IF IT IS NOT FEASIBLE TO DIVERT UP-SLOPE DRAINAGE AROUND A  PROJECT AREA,  A CHANNEL OR OTHER CONVEYANCE STRUCTURE MAY BE USED TO TRANSPORT WATER THROUGH A PROJECT AREA.

DIVERSIONS AND CONVEYANCE CHANNELS SHALL BE PROTECTED FROM ACCELERATED EROSION AND SEDIMENT POLLUTION THROUGH THE DEVELOPMENT AND IMPLEMENTATION OF BEST MANAGEMENT  PRACTICES.

OUTLET STRUCTURES .  

 (i) SUITABLE OUTLET PROTECTION MUST BE PROVIDED UNLESS A DISCHARGE VELOCITY OF LESS THAN 3.0 FPS (0.9 METERS PER SECOND) CAN BE MAINTAINED. 

(ii)  ALL OUTLET STRUCTURES SHALL BE STABILIZED BEFORE USE. 

[(d)] (10) [Sedimentation] SEDIMENT basins. [The following applies to sedimentation basins.] 

[(1)] (i)  A [sedimentation] SEDIMENT  basin shall have [a capacity of 7,000] 5,000 cubic feet [for each acre of  project area tributary to it and shall be provided with a 24-inch freeboard.] OF  STORAGE  FOR  EACH ACRE  (495 CUBIC METERS FOR EACH HECTARE) TRIBUARY TO THE BASIN,  PLUS AN ADDITIONAL 2,000 CUBIC FEET OF  SEDIMENT STORAGE  CAPACITY FOR EACH DISTURBED ACRE  (142.5 CUBIC METERS FOR EACH HECTARE) TRIBUTARY TO THE BASIN.   THE MINIMUM STORAGE  ELEVATION SHALL BE DETERMINED BY THE  TOTAL STORAGE CAPACITY  OF THE BASIN.  THE  MINIMUM  SEDIMENT  STORAGE  ELEVATION  SHALL BE  DETERMINED BY THE SEDIMENT  STORAGE  CAPACITY OF THE BASIN.

 [(2)] (ii)  The basin shall be cleaned [when] BEFORE  the ACCUMULATED SEDIMENT EXCEEDS THE SEDIMENT storage [capacity] ELEVATION  of the basin . [is reduced to 5,000 cubic feet per acre of project area tributary to it.] 

[(3)] (iii) Outlet structures shall be designed to pass a minimum flow of 2 cubic feet per second (CFS) (0.06 CUBIC METERS PER SECOND)  for [each acre of project area] FOR THE TOTAL DRAINAGE AREA tributary  to the basin.  

(A)  THE  ENTIRE  2 CFS (0.06 CUBIC METERS PER SECOND) DISCHARGE CAPACITY MUST BE PROVIDED ABOVE THE MINIMUM STORAGE ELEVATION.  

(B)  A MINIMUM  OF  24 INCHES (61 CENTIMETERS) OF FREEBOARD MUST BE PROVIDED ABOVE THE ELEVATION OF THE 2 CFS (0.06 CUBIC METERS PER SECOND) DISCHARGE CAPACITY. 

(C)  EVERY SEDIMENT BASIN SHALL BE PROVIDED WITH AN EMERGENCY SPILLWAY.

[(4)] (iv) The discharge from a [sedimentation] SEDIMENT basin shall be to a [natural waterway] WATER OF THE COMMONWEALTH CAPABLE OF RECEIVING THE DISCHARGE WITHOUT CAUSING ACCELERATED EROSION OR SEDIMENT POLLUTION. 

[(5)] (v) [Sedimentation] SEDIMENT basins shall be structurally sound and protected from unauthorized acts of third parties. 

(vi)  IN ADDITION TO THE REQUIREMENTS OF THIS SECTION, SEDIMENT BASINS REQUIRING PERMITS UNDER THE DAM SAFETY AND ENCROACHMENT ACT (32 P.S. §§ 693.1 ET SEQ) ARE REQUIRED TO MEET THE DESIGN, CONSTRUCTION, OPERATION AND MAINTENANCE AND OTHER PERMIT REQUIREMENTS OF TITLE 25, CHAPTER 105 (RELATING TO DAM SAFETY AND WATERWAY MANAGEMENT).

 (11)  SEDIMENT TRAPS.

SEDIMENT  TRAPS SHALL BE DESIGNED TO HAVE  A CAPACITY OF 2,000 CUBIC FEET OF STORAGE FOR EACH ACRE  (142.5 CUBIC METERS PER HECTARE) TRIBUTARY TO IT.

THE MAXIMUM DRAINAGE AREA TO A SEDIMENT TRAP IS FIVE ACRES ( 2 HECTARES).

OUTLETS FROM SEDIMENT TRAPS SHALL BE STRUCTUALLY SOUND.

THE DISCHARGE FROM A SEDIMENT TRAP SHALL BE TO A WATER OF THE COMMONWEALTH CAPABLE OF RECEIVING THE DISCHARGE WITHOUT CAUSING ACCELERATED EROSION OR SEDIMENT POLLUTION

§102.13.  RESERVED.

[RESTORATION]  PERMANENT STABILIZATION 

§ 102.21. [Applicability] RESERVED. 

[Sections 102.21–102.24 apply to earthmoving activities which have not been stabilized.] 

§ 102.22. PERMANENT Stabilization. 

 [Upon completion of the project ] IN ACCORDANCE  WITH § 102.12 (4),  ALL DISTURBED areas [disturbed by the project] shall be PERMANENTLY stabilized so that accelerated erosion WILL BE MINIMIZED AND SEDIMENT POLLUTION shall be prevented.  FOR A PROJECT TO BE CONSIDERED PERMANENTLY STABILIZED, ALL DISTURBED AREAS MUST BE COVERED WITH:

A MINIMUM OF  70%  PERRENNIAL VEGETATIVE COVER OF UNIFORM COVERAGE AND DENSITY; OR

AN ACCEPTABLE BEST MANAGEMENT PRACTICE  WHICH  PERMANENTLY MINIMIZES ACCELERATED EROSION.

  § 102.23. [Interim control measures] RESERVED. 

[An erosion and sedimentation control facility required or necessary to protect areas from erosion during the stabilization period shall be maintained until stabilization is completed.] 

§ 102.24. [Final measures]  REMOVAL OF BEST MANAGEMENT PRACTICES. 

Upon completion of PERMANENT stabilization, ALL [unnecessary or unusable control facilities] BEST MANAGEMENT PRACTICES shall be removed, the areas shall be graded and the [soils] DISTURBED AREAS shall be stabilized[.], UNLESS OTHERWISE AUTHORIZED BY THE DEPARTMENT OR ITS DESIGNEE.  

PERMITS [AND PLANS]

§ 102.31. Permit requirements. 

(a) EXCEPT AS SET FORTH IN SUBSECTION 102.31(b), [A] a person or municipality who engages in an [earthmoving] EARTH DISTURBANCE activity CONSISTING OF FIVE OR MORE ACRES ( 2  OR MORE  HECTARES) OF DISTURBANCE OVER THE DURATION OF THE PROJECT [within this Commonwealth] shall obtain a permit prior to commencement of the activity. [ except a permit may not be required under the following circumstances: 

(1) If the earthmoving activity involves plowing or tilling for agricultural purposes. 

(2) If an activity is required to obtain a permit under The Clean Streams Law (35 P. S. §§ 691.1–691.1001), the Surface Mining Conservation and Reclamation Act (52 P. S. §§ 1396.1–1396.21), the Water Obstruction Act (32 P. S. §§ 681–691) or Chapters 91–95, 97 and 101. 

(3) If an earthmoving activity disturbs less than 25 acres. 

If an activity involving more than 25 acres is subdivided into parcels of less than 25 acres and earthmoving is undertaken on noncontiguous parcels and the parcels are stabilized before contiguous parcels are disturbed. ]

THE FOLLOWING EARTH DISTURBANCE ACTIVITIES REQUIRE THE DEVELOPMENT OF AN EROSION AND SEDIMENT CONTROL PLAN, BUT  DO NOT REQUIRE A PERMIT:

AGRICULTURAL PLOWING AND TILLING;

TIMBER HARVESTING  ACTIVITIES;  

HAVING PROJECT AREAS OF LESS THAN 250 ACRES (101 HECTARES) OR

HAVING PROJECT AREAS OF 250 OR MORE ACRES (101 OR MORE HECTARES)  WHEN THE EROSION AND SEDIMENT CONTROL PLAN HAS BEEN REVIEWED AND APPROVED BY THE DEPARTMENT OR DELEGATED COUNTY OR OTHER UNIT OF LOCAL GOVERNMENT.

FEDERAL, STATE, AND MUNICIPAL ROAD MAINTENANCE ACTIVITIES; 

 [(b)]  (c) The Department, after publication in the Pennsylvania Bulletin, may reduce the acreage limitation set forth in [subsection (a)(3) or (4)] SECTION 102.31 (a), on the following basis: 

(1) On a Statewide basis at its discretion.

(2) For special areas where the Department deems it necessary. 

(3) For a county or municipality within this Commonwealth. 

[(c)] (d)  Even though an activity is not required to obtain a permit under the exceptions set forth in this section, the person or municipality undertaking the activity shall comply with the other provisions of this chapter. 



§ 102.32. Application for permit.

(a)  APPLICATIONS FOR NATIONAL POLLUTANT DISCHARGE ELIMINATION SYSTEM (NPDES) PERMITS SHALL BE SUBMITTED PURSUANT TO REQUIREMENTS CONTAINED IN CHAPTER 92 OF THIS TITLE.

(b) FOR ACTIVITIES OTHER THAN THOSE IN § 102.32(a) , WHICH REQUIRE  PERMITS PURSUANT TO § 102.31, THE FOLLOWING SHALL APPLY: 

    [(a)] (1)  Applications for permits shall be submitted by the person or municipality [undertaking] ENGAGING IN  the [earthmoving] EARTH DISTURBANCE activity. [In the case of land development, the application shall be submitted by the land developer rather than the contractor or agent..] 

[(b)] (2)  Applications shall be accompanied by an erosion and [sedimentation] SEDIMENT control  plan and other documents the Department may require. 

    [(c)] (3)  Applications shall be accompanied by [a processing] AN APPLICATION  fee of [$200] $500 . 

RESPONSIBILITIES OF LOCAL GOVERNING BODIES 

§ 102.41. Administration by local governing bodies. 

(a) The Department may, at its discretion, delegate BY WRITTEN AGREEMENT the administration and enforcement of this chapter to counties, INCLUDING COUNTY CONSERVATION DISTRICTS, [and other units of local government provided the county or other unit of local government has and implements an acceptable plan approved by the Department for administering such a program. ] AND OTHER UNITS OF GOVERNMENT  PROVIDED THAT THE  DESIGNEE  HAS AND IMPLEMENTS  A PROGRAM IDENTIFIED IN THE DELEGATION AGREEMENT.   

(b) An acceptable [plan] PROGRAM shall include adequate and qualified staff for the review of erosion and sediment control plans and for  the [surveillance] COMPLIANCE and enforcement of the provisions of this chapter.   An acceptable plan shall have the concurrence and approval of the [Commissioners] GOVERNING BODY of the county in which the local unit of government OR COUNTY CONSERVATION  DISTRICT operates. 

(c) The Department will retain program administration over projects which cross the political boundaries of local governing bodies who have been delegated the administration of the provisions of this chapter[.] UNLESS OTHERWISE  SPECIFIED IN THE DELEGATION AGREEMNT 

§ 102.42. Notification of application for [building] permits. 

A [local governing body] MUNICIPALITY OR COUNTY  which issues building OR OTHER LAND DEVELOPMENT  permits shall notify the Department or its designee [immediately upon] WITHIN 5 DAYS OF  receipt of an application for [a building, the] A permit involving an [earthmoving] EARTH DISTURBANCE  activity [which affects] CONSISTING OF  5 acres ( 2 HECTARES) or more of land. 

§ 102.43. Withholding building permits. 

A [local governing body ] MUNICIPALITY OR COUNTY may not issue a building  OR OTHER LAND DEVELOPMENT permit to those [engaged in earthmoving] PROPOSING OR ENGAGING IN EARTH DISTURBANCE activities requiring a Department permit until the Department OR DELEGATED COUNTY OR OTHER UNIT OF LOCAL GOVERNMENT  has issued the permit under §§ 102.31 and 102.32 (relating to permit requirements; and application for permit). 





IMPLEMENTATION 

§ 102.51.[ Effective dates] RESERVED. 

[(a) This chapter became effective 30 days after its adoption by the EQB except §§ 102.31 and 102.32 (relating to permit requirements; and application for permit), which requires permits prior to the commencement of an activity, became effective on July 1, 1973, and § 102.4 (relating to general), which require preparation of erosion and sedimentation control plans, shall become effective according to the following schedule: 

(1) Agricultural activities, plowing and tilling only–July 1, 1977. A person or municipality who has applied to a conservation district for an erosion and sedimentation control plan or a conservation plan before July 1, 1977, shall be considered in compliance with the deadline date. 

(2) Existing earthmoving activities–January 1, 1974. 

(3) New earthmoving activities started after adoption of this chapter but before July 1, 1973–July 1, 1973. 

(4) New earthmoving activities started after July 1, 1973–prior to commencement of the activity. 

(b) The Department, if it finds that it is in the best interest of the Commonwealth, may order the development and implementation of erosion and sedimentation control plans or require permits sooner than the dates set forth in this section.]





DRAFT 03/26/27  3:21 PM



�PAGE  �





                                                                                                   � PAGE �1�








